# Chapter 4: DC-DC CONVERTER

## BCL50 series Power System block diagram

BCL50 series Power System block diagram

### Description

This specification defines the performance and characteristic of 60W AC adapter power supply. It supplies a constant voltage 19V output source for BCL50 series notebook computer.

### Feature

Accepts universal input from 90V AC to 264V AC

- Offers constant Voltage 19V output source with 60W max output power capacity.

- High efficiency 83% min at 115Vac

- Compact Size

### Electrical Specification

- Input Voltage range:	universal input, 90VAC TO 264AC

- Inrush current:	No damage shall occur and the input fuse shall not blow

- Input frequency range:	47~63Hz

- Input Current:	1.8Amax at 100VAC
	0.9Amax at 240 VAC

- Start-up time:	 3sec Max. @115Vac

- HOLD-UP time:	5ms min. @115VAC,  full load condition

- OVP: 	27V max. automatic shut down

- Short circuit protection: Output can be shorted without damage, and auto recovery

- OUTPUT Voltage Regulation: 18.5-20V including the effects of line Voltage variation, load current, ripple and noise

- OUTPUT Current: 0Amin, 3.16Amax continuous

- OUTPUT Voltage ripple: 300mv PK-PK for resistor load

- OUTPUT Voltage Dynamic regulation: Output voltage within 18.5V~20V, load current 10%100%, frequency 100Hz, 50% duty cycle.

- 2.5.3.14  DC OUTPUT PIN OUT:

PIN1	Center Pin Adapter +output

PIN2	Barrel (Ring) Adapter returns.

- 2.5.3.15  POWER CORD: 2PIN LP5*1.8M*LS7C US 2PL

2PIN LP21*1.8M*LS7E US EC 2PL(EC)

- 2.5.3.16  Temperature Range:

Operating temperature:	0 °C TO 40 °C

Storage temperature:	-20 °C TO 65 °C

CAUTION

Danger of explosion if battery is incorrectly replaced.

Replace only with the same or equivalent type recommended by the manufacturer.

Dispose of used batteries according to the manufacturer’s instructions.

# DC-DC CONVERTER

## Description

The DC-DC converter is designed to supply the power for ACL10 series notebook computer of Compal. It supply +5VALWP, +3VALWP, +12VALWP, +1.8VSP, +1.05VALWP,+1.5VALWP ,+1.2VSP,1.25VSP,2.5VP for logical system, +CPU_CORE, for CPU and supplies +3VALWP for the built-in NS87591 microprocessor which handles the keyboard and PMU control functions of the system. The power ON/OFF is controlled by NS87591. There is also a built-in charger power source. It can charge battery pack whether the computer is ON or OFF.

- Features

- 2.6.2.1  High efficiency, up to 85% (using battery)

- 2.6.2.2  Accept wide range DC input voltage from 12V to 20V

- 2.6.2.3  Built-in charger power source

- 2.6.2.4  The power ON/OFF is controlled by software

- Electrical specification

- Input Voltage/Current

12V to 20V at the summing point of AC-DC and battery

INPUT Current 5.1A max from battery

INPUT Current 3.16A max from AC-DC Adapter.

Temperature Range:

- 2.6.4.1  Operating temperature : 0° C to 40° C

- 2.6.4.2  storage temperature range : -20° C to 65° C

## DC/DC OUTPUT

### Fixed output voltage/Current

Item	+5VALWP	+1.8VALWP	+CPU_COREP +1.2VSP

nominal voltage	+5V	+1.8V	+1.3V                +1.2V

min. current	0A	0A	0A                       0A

max. current	3.5A	1.5A	30A                     0.1A

peak current	4.5A	2.5A	40A                     0.3A

total regulation 	5V±5%	1.8V±5%	1.3±0.1V            1.2V+5%

ripple voltage	100mVp-p max.	100mVp-p max.	50mVp-p max   60mVp-p max

Item	+12VALWP	+3VALWP	+1.5VALWP	+2.5VP

nominal voltage	+12V	+3.3V	+1.5V	+2.5V

min. current	0A	0A	0A	0A

max. current	100mA	4A	1.0A	5A

peak current	100mA	5A	1.5A	6A

total regulation	12V±5%	3.3V±5%	1.5V±5%	+1.25V±5%

ripple voltage	200mVp-p max.	100mVp-p max.	100mVp-p max.	50mVp-p max.

Item	+1.25VSP        +1.8VSP                +1.05VALWP

nominal voltage	+1.25V             +1.8V                    +1.05V

min. current	0A                      0A                           0A

max. current	2A                      1.5A                        1A

peak current	3A                      2.5A                         2A

total regulation	1.25V±2%         1.8V±5%                 1.05V±5%

ripple voltage	50mVp-p max.  50mVp-p max.         50mVp-p max

- Charger

- 2.6.6.1  Controlled by NS87591 microprocessor from motherboard

- 2.6.6.2  Temperature sense capability for the battery (charge active between 0°C~ 50°C)

- 2.6.6.3  Fast charge 3.0Amps(typical.) for Li-Ion Battery when system off, approach 20W fast charge when system ON. (depend on system load)

- 2.6.6.4  When system turn off, the charged time from empty to full be 3.0 hrs typically for Li-ION battery.

- 2.6.6.5  Other battery services are presented by NS87591 microprocessor includes maximum charging timer, charging temperature range etc.

- 2.6.6.6  Charger power:

- BATT+ Constant voltage mode: 16.8V±1% for Li-Ion battery.

- Constant current mode: 3.0A±8%

- Constant adapter current mode: 3.3A±6%

### OVER Current protection:

+5VALWP:	6.6~10.5A
+3VALWP:	6.4~10.4A

+CPU_COREP:	30A~35A

### OVER Voltage protection:

+5VALWP:	5.51 ~6.00V

+3VALWP:	3.53~ 3.84V

+CPU_COREP	1.85 ~2.1V

### Under voltage protection

+5VALWP:	5V *(80%)

+3VALWP:	3.3V *(80%)

+CPU_COREP	+CPU_COREP Voltage * (65 ~75) %

### Short circuit protection:

Latch mode for +5VALWP, +3VALWP, +CPU_COREP

## I/O

### P1:AC adapter input Jack socket

Pin 1: Center pin Adapter power + input

Pin 2: Barrel (Ring) Adapter power return

### DC/DC interface

| Item | Description | I/O |
| --- | --- | --- |
| SUSP# | active low input, enable skip mode of TPS5120 | I |
| 51ON# | active low input, this pin controls VS output when system is using battery power, it is controlled by NS87591. | I |
| FSTCHG | active high input, when NS87591 judges the battery shall be charged, then this pin will be pulled high. | I |
| IREF | Battery change current control IREF= 1.31 * I change ,IREF= 0 ~ 3.3V | I |
| ACIN | active high output, when adapter present, this pin shall be pulled high, output for NS87591. | O |
| VGATE | power good output signal, active high, from +CPU_COREP turn on edge and goes low before falling –5% of +CPU_COREP. | O |
| VR_ON | Active high input, +VID(1.2V) on/off pin. | I |
| BATT_TEMPA | An analog signal, it offers the status of battery temperature for NS87591. | O |
| VID [0..5] | CPU VID Pin. It determinate the +CPU_COREP output voltage. | I |
| 6C/8C# | High:6 Cell Li-ION Battery, Low: 8 Cell Li-ION Battery | O |
| AB/I | Battery discharge active pin. | O |
| EC_SMD1 | Main battery SMBus data pin. | I/O |
| EC_SMC1 | Main battery SMBus clock pin. | I/O |
| MAINPWON | H/W Thermal protection | O |
| STP_CPU# | CPU MODE control pin , It comes from sorth bright | I |
| PM_DPRSLPVR | CPU MODE control pin , It comes from sorth bright | I |
| PSI# | CPU MODE control pin , It comes from sorth bright | I |

## BATTERY

### Li-ion Smart battery for BCL50 series

18650 2P4S 14.8V/4300mAH, Li-ion battery

More than 300 charging/discharging cycles.

Modularized battery pack, easy to be replaced.

### On board RTC battery:Maxell ML1220T13RE /3.0V /14mAH Lithium.

### Sanyo ML1220T28 /3.0V /15mAH Lithium

## BCL50 INVERTER SPECIFICATION

### Description

This inverter is designed to light up the CCFL of LCD for notebook.It should be supported following LCD panels.

- BCL50 project

| No. | Supplier | Model | Type | Frequency (KHz) |  |  | Current (mA) |  |  | VS max. (at 0oC) (Vrms) | Work Voltage (Vrms) | Brightness (cd/m2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | min | typ | max | min | typ | max |  |  |  |
| 1. | AU | B141XN04 | 14.1” XGA | 50 | 60 | 80 | 3 | 6 | 6.5 | 1530 | 660 at 6mA | 150 at 6mA |
| 2. | CPT | CLAA141XF01 | 14.1” XGA | -- | 50 | -- | -- | 5.5 | 7 | 1340 | 680 at 5.5mA | 150 at 5.5mA |
| 3. | CMO | N141X6-L01 | 14.1” XGA | 45 | 60 | 80 | 2 | 6 | 7 | 1130 | 650 at 6mA | 150 at 6mA |
| 4. | SANYO | TM150XG-02L11 | 15.0”XGA | 40 | -- | 65 | 3 | -- | 6 | 1550 | 700 at 5mA | 150 at TBDmA |
| 5. | LG | LP150X05 | 15.0” XGA | 45 | 58 | 80 | 3 | 5 | 6 | 1370 | 685 at 5mA | 160 at 6mA |
| 6. | AU | B150PG01 | 15.0” SXGA+ | 50 | 60 | 70 | 3 | 5.5 | 7 | 1150 | 700 at 5.5mA | 150 at5.5mA |
| 7. | Hitachi | TX38D91VC1FAC | 15.0”SXGA+ | 40 | 50 | 70 | 4 | -- | 6 | 1600 | 690Vrms | 150 at TBDmA |

There are two control signals that come from system to control lamp brightness. One signal is named DAC_BRIG, which limits current to meet LCD lamp current specification. Another one is named PWM, which adjusts lamp brightness. This inverter brightness is adjusted by PWM burst mode. The PWM burst mode is that turning on and off the lamp at a rate of 150Hz. The effective brightness is a function of the duty cycle.

### Features

- 2.7.2.1  Wide range 9V to 21V input voltage

- 2.7.2.2  Brightness adjustment by PWM burst mode.

- 2.7.2.3   Close loop controls lamp current.

### Absolute maximum rating

- 2.7.3.1  Environment: Temperature: Operating temperature: 0℃ ~ 55℃

Storage temperature : -20℃ ~ 70℃

Humidity:  0 ~ 90% without condensation

MTBF:  MIN 50000 hours.

### Electrical characteristic

| No | Item | Symbol | Min. | Typ. | Max. | Unit | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Input voltage | INV_PWR | 9 | 14.8 | 21 | V | Lamp can be ignited by 7.5V input. *Note 1 |
| 2 | Input current | Iin | -- | 0.33 | -- | A |  |
| 3 | Lamp current | IL | 3.0 | -- | 6.8 | mA | DAC=0V *Note 2 |
| 4 | Lamp current | IL | 2.7 | -- | 6.3 | mA | DAC=1V |
| 4 | Frequency | F | 45 | 55 | 65 | KHz | *Note3 |
| 5 | Output power | Pout | -- | -- | 4.5 | W |  |
| 6 | Efficiency |  | 80% | -- | -- | -- |  |
| 7 | OP-Lamp voltage | Vs | 1600 | -- | -- | V | At 0oC |
| 8 | Starting time | Tvs | 1 | -- | 1.5 | Sec |  |
| 9 | Dispoff# |  | 2.8 | 3.3 | 3.6 | V | Backlight on/off signal |
|  |  |  | 0 | 0.5 | 0.8 | V | Low level |
| 10 | Limited lamp maximum current | DAC-BRIG | 0 |  | 3.3 | V | *Note 2 |
| 11 | PWM signal *note 4 | INV_PWM | 142 | 150 | 158 | Hz | PWM signal frequency |
|  |  |  | 3.0 | 3.3 | 3.6 | V | PWM signal amplitude |
|  |  |  | 30 | -- | 100 | % | Duty=TonPeriod |
| 12 | lamp current over-shoot | IZero−PK | -- | -- | 10 | % | Line transient( 10.8V to 21V/100us) and turn on transient |
| 13 | Current Waveform factor | IpIrms | 1.27 | 2 | 1.56 | Multiple | OR I−pIrms *10 |
| 14 | Unbalance Rate | Ip−I−pIrms | -10% | 0 | +10% | Multiple |  |
| 15 | Turn off current (Hight side) | IHL | -- | -- | 0 | A | PWM=30% |
| 15 | Turn off voltage (Low side) | Voff | -- | -- | 150Vp-p | V | PWM=30% |
| 16 | Voltage Rise time (Low side) | Trise | -- | -- | 300us | us | PWM=30% |
| 17 | Voltage fall time(Low side) | Tfall | -- | -- | 300us | us | PWM=30% |

Notes:

*1. The lamps can be ignited by 7.5V input voltage of inverter .But ,once the inverter input is 7.5V, its performance may not fully meet the electronic spec. defined.

*2. Limited lamp maximum current by DAC_BRIG signal:

When DAC_BRIG voltage is 0V and INV_PWM enables (100%), lamp has max. limited current.

When DAC_BRIG voltage is 3.3V and INV_PWM enables (100%), lamp has min. limited current.

When add 1V DAC, the 100% Lamp current will decrease 0.5mA.

DAC_BRIG signal comes from system chipset with internal resistance of 3KΩ.

*3. Inverter operating frequency should be within specification (45~65kHz) at max. and min. brightness load.

*4. INV_PWM enable implies INV_PWM signal is High level (On duty cycle is 100%). It is a square wave of 150Hz to adjust backlight brightness that is a function of PWM duty cycle. Backlight brightness is maximum value under INV_PWM at 100% and brightness is minimum under INV_PWM at 30%.

*5. The system interface signals belong to 3.3V.

*6. Please make sure open lamp output voltage should be within starting voltage specification.

*7. Inverter should pass human body safety test.

*8. Inverter should no smoking by any component open/ short test

*9. Transformer voltage stress should not be over 85% under any condition

( turn on overshoot transient and line transient).

*10. Audio noise should be less than 36dB at 10 cm distance.

# Electrical specification

### Electrical specification

| No | Symbol | Min. | Typ. | Max. | Unit | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Vopen. | -- | 650 | -- | Vrms | Lamp operating voltage(650+/-50) |
|  | IL | 6.2 | 6.5 | 6.8 | mArms | DAC_BRIG: 0 V, PWM: 100% |
|  | IL | 3.0 | 3.3 | 3.6 | mArms | DAC_BRIG: 0 V, PWM:30% |
|  | IL | 5.7 | 6 | 6.3 | mArms | DAC_BRIG: 1 V, PWM: 100% |
|  | IL | 2.7 | 3 | 3.3 | mArms | DAC_BRIG: 1 V, PWM:30% |
|  | f | 45 | 55 | 65 | KHz |  |
|  |  | 80% | -- | -- | -- |  |

### Thermal

All components on inverter board should follow below rules:

Component using conditions (component stress) must be within component specification including voltage rating, current rating, temperature etc.

### Connector description

- 2.7.5.1 Input Connector:

MOLEX:52207-0990 ; JST SM07B-SRSS-TB

| Pin No. | Symbol | Description |
| --- | --- | --- |
| 1 | INV_PWR | Input voltage |
| 2 | INV_PWR | Input voltage |
| 3 | NC | No connection |
| 4 | INV_PWM | Adjust brightness by burst mode |
| 5 | DISOFF # | Backlight on/off control, active HIGH |
| 6 | DAC_BRIG | Max. current limit |
| 7 | NC | No connection |
| 8 | GND | Power system return |
| 9 | GND | Power system return |

- 2.7.5.2 Output Connector:

CN2: JST_SM02B_BHSS-1

| Pin No. | Symbol | Description |
| --- | --- | --- |
| 1 | HV | Connected to high voltage of LCD lamp |
| 2 | LV | Connected to low voltage of LCD lamp |

Note : Please mark “ CAUTION HIGH VOLTAGE” around CN2

### Safety Protection

- 2.7.6.1 Open lamp protection:

When inverter is on open lamp status, any component on inverter should be O.K and inverter is no damaged, no fire and no arcing. If inverter can’t shunt down during open lamp happen, inverter must pass below conditions:

- Human body test.

- Open lamp burning: Inverter burns for 24 hours at open lamp status. No parts damage.

### Human body safety test:

Short inverter output, transformer secondary output to GND by a 2KΩ resistor which connects one end to GND and another one to those outputs. They should meet output current limitation requirement as follow. Output current I is the current that flows through 2KΩ resistor.

- Output current  I ≦ 0.7mA , if frequency f ≦ 1KHz

- Output current  I ≦ 0.7mA * f (kHz) , if f ≧ 1KHz.

However, output current should be less than 70mA even frequency is more than 100KHz.

### Abnormal test:

Any one component is short or open; inverter should be no fire, no arcing. And result must meet output current limitation requirement.

### PCB drawing