# Software Specification For System BIOS

System Component Summary

Hot Keys

| Fn +  | Decrease brightness |
| --- | --- |
| Fn +  | Increase brightness |
| Fn + F5 | LCD/Monitor(CRT)/TV switch When pressing the hot key, the display device will switch among LCD only, LCD+CRT, CRT only, LCD+TV and TV only under WINDOWS. It doesn’t support switching to TV under pure DOS mode. |

### Fn+F5 Toggle Display Sequence:

LCD  CRT  LCD+CRT  TV  LCD+TV

Connect LCD+CRT+TV at same time

 Boot with LCD+CRT+TV first time, LCD+CRT show display at same time.

 Toggle display by Fn+F5, the display sequence is LCD  CRT  LCD+CRT

 TV LCD+TV.

Connect LCD+CRT at same time

 Boot with LCD+CRT first time, LCD+CRT show display at same time.

 Toggle display by Fn+F5, the display sequence is LCD  CRT  LCD+CRT.

Connect LCD+TV at same time

 Boot with LCD+TV first time, LCD show display only.

 Toggle display by Fn+F5, the display sequence is LCD  TV  LCD+TV.

## Buttons

Power Button

The activity of the power button is as follows:

 If power button is pressed for less than 100 will seconds then nothing happens.

 If power button is pressed for more than 100 will seconds but less than 4 seconds then system would execute User Requested OFF.

 If power button is pressed for more than 4 seconds then the notebook will be powered off by power button over-ride feature.

 If OS is running in ACPI mode, the power button acts as the sleep button, and let OS controls the policy of power button (shout down, standby or hibernate).

### Power Button Over-ride

Holding down the Power Button for 4 seconds will cause an unconditional transfer to the off state without notifying the operating system.

### Lid switch

This section describes the expected behavior of the system when the lid is opened or closed by the user.

 If the system is running under legacy mode:

Closing the lid will turn off LCD backlight.

 If the system is running under ACPI mode:

The operating system will determine what action to take when the lid is opened and closed.

The possible actions of lid close are None, Standby, Hibernate.

### System status indicators

Please refer to Keyboard BIOS specification.

## Core BIOS Features

Enhanced IDE Disk Drive Support (EDD)

In addition to AT standard disk drive support:

 Auto detecting and sizing of all IDE drives.

 Logical Block Addressing

 Fast DMA support

 Ultra DMA100/66/33 support

 S.M.A.R.T

### Multi Boot

The notebook can support Multi-Boot for selecting the boot sequence of hard disk, USB floppy, CD-ROM, Network and USB ATA device in Setup.

The notebook doesn’t support booting from legacy  floppy.

### Quiet Boot

Quiet Boot replaces the customary technical messages during POST with a more visually pleasing and comfortable display (OEM screen).  During POST, right after the initialization of VGA, The notebook displays an illustration called the OEM screen during system boot instead of the traditional POST screen that displays the normal diagnostic messages.

The OEM screen stays up until just before the operating system loads unless:

 Pressing <Esc> to switch to the POST screen and the boot process will continue until the end of POST.

 Pressing <F2> to enter Setup.

 Whenever POST detects a non-terminal error, it switches to the POST screen near the end of POST, just prior to prompting for a password.

 If the BIOS or an option ROM request keyboard input, the system switches over to the POST screen with prompts for entering the information. POST continues from there with the regular POST screen.

### Boot Block

The Flash ROM used in many systems today offer the customer the advantage of electronically reprogramming the BIOS without physically replacing the BIOS ROM. This advantage, however, does create a possible hazard: power failures or fluctuations that occur during updating the Flash ROM can damage the BIOS code, making the system unbootable.  To prevent this possible hazard, many Flash ROM include a special non-volatile region that can never be erased. This region, called the boot block, contains a fail-safe recovery routine. If the boot block finds corrupted BIOS, it prompts the end user to insert a diskette, from which it loads several files that replace the corrupted BIOS on the Flash ROM with an uncorrupted one.

### New Interrupt 15h extensions

The BIOS must support the recently defined standard INT 15 extensions:

### Big Memory

- Big memory support can report greater than 64 megabytes of RAM.  The notebook supports the INT 15h big-memory reporting functions of E801h, E881h, and E820h.  This feature reports all available extended memory (both below and above the 64MB limit) using both a real mode (E801h) and a 32-bit protected mode (E881h) interface. Operating systems can access the real-mode interface through the standard INT 15h call.  They can access the protected-mode interface through a 32-bit interrupt call, much like the EISA protected-mode interface. The Microsoft-defined E820h function returns a complete memory map through a series of repeated calls.

### Plug-n-Play (PnP) Support

- To achieve the goal of PnP, a POST conflict detection and resolution (CDR) module, and a run-time services module will be integrated into the system BIOS.

- The PnP runtime service module includes multiple interfaces so that the system can support the current DOS, as well as Win98 operating system that include specific support for the PnP BIOS specification.

## Security Features

The security feature to be supported is password.

### 2 Level Passwords

The notebook supports two levels password protection. The password support consists of a User Password and an Supervisor Password.  They each contain up to eight characters, and are stored in CMOS.

When the password is enabled, the notebook may display a suitable password prompt on the main display in the following situations:

 Turning on from the Off State.

 Turning on from Hibernate.

 Entering to Setup.

The User will attempt to enter a password, then press ENTER If the User fails to enter the password in three tries the system will return to the state it was turned on from (off, Suspend to Disk).

All user data on screen must not be visible before entering the correct password.

### User Password

The user can choose to enable or disable:

 Password required on boot

 Password required on resume from hibernate

The user password may not be set unless the administrator password is set.  If the user wishes to only have one password then the administrator password is used.

### Supervisor Password

There are three primary uses for the Supervisor Password:

 To protect the contents of the PC Identification strings from changes by the user.

 Protect users from changing system configuration that could cause the notebook to malfunction.

- As the users password if only a single password is desired.

### Valid Password Characters

Valid Password Characters

| Symbol Character | Symbol Name |
| --- | --- |
| A-Z | Alphabets A through Z (Not Case Sensitive) |
| 0-9 | Numerical Characters. |
| - | Dash |
| = | Equal Sign |
| [ | Left Bracket |
| ] | Right Bracket |
| . | Period |
| , | Comma |
| ; | Semi-colon |
| / | Slash |
| \ | Back-slash |

The password is stored as scan codes.

Both passwords will be encrypted before being stored in CMOS RAM using the standard Phoenix password encryption technique.

## Thermal management

Please refer to Keyboard BIOS specification.

## Power Management

### Introduction

The notebook supports ACPI. The system will dynamically switch to ACPI mode for configuration and power management when an ACPI OS is loaded.

When ACPI is not loaded and enabled, the power management function will be disabled.

### System Time-outs

If the system is running in ACPI mode, system Time-outs is handled by the operating system.  BIOS time-outs are disabled. System time-outs are set using the control panel power applet.

### System Power Management

The overall system can be in one of the system power states as described below:

| ACPI mode | Power Management |
| --- | --- |
| Mech. Off (G3) | All devices in the system are turned off completely. |
| Soft Off (G2/S5) | OS initiated shutdown. All devices in the system are turned off completely. |
| Working (G0/S0) | Individual devices such as the CPU and hard disk may be power managed in this state. |
| S1 Sleeping State |  CPU Stop Clock VGA Standby  turn off back-light  PCMCIA Standby  Audio Standby ,  Hard Disk Spin Down motor  CD-ROM Spin Down  Super I/O Power down |
| S3 Sleeping State |  CPU set power down  VGA Suspend  PCMCIA Suspend  Audio Suspend  Hard Disk Power Down  CD-ROM Power Down  Super I/O Power Down |
| S4 Sleeping State | System Saves all system states and data onto disk prior to power off the whole system. |

### Device Power Management

Under ACPI mode, the device specific power management supported by this notebook includes the CPU throttling, monitor power management and the hard disk.

### CPU power management

Support CPU low power states C3,C4 and SpeedStep.

### Hard Disk

The hard disk will be spun down after a period of no activity based on the settings of the OS.

### Display Device

The monitor can be turned off after a period of no activity based on the settings of the OS.

### System Wake Up Sources

The table below lists the wake up events for all low power states:

| Events | S1 | S3 | S4 | S5 | Process required |
| --- | --- | --- | --- | --- | --- |
| Hot key*1 | X | X | X | X | X |
| Any Key | X | O | X | X | X |
| Power button | O | O | O | O | X |
| Lid close | X | X | X | X | X |
| Lid open | X | X | X | X | X |
| Modem Ring (PCMCIA Modem) | X | X | X | X | X |
| Modem Ring (MiniPCI Modem) | O | O | X | X | X |
| Modem Ring (USB Modem) | X | X | X | X | X |
| LAN (PCMCIA) | X | X | X | X | X |
| LAN (MiniPCI / onboard) | O | O | O | O | X |
| LAN (USB) | X | X | X | X | X |
| AC/Battery | X | X | X | X | O |
| Thermal | X | X | X | X | O |
| RTC | O | O | X | X | X |
| LPT/KB/Mouse/FDD/HDD | X | X | X | X | X |
| Audio/Video activity | X | X | X | X | X |
| PCMCIA | O | O | X | X | Driver |
| USB | *2X | *2X | X | X | Driver |
| CRT (no event) plug/unplug | X | X | X | X | O |
| Power Kill (no event) | X | X | X | X | KB only |
| Critical low battery | X | X | X | X | X |

Field ‘Process Required’ identifies that further process for the occurred events must be processed during wake up or resume procedure.

Notes:

-  Hot keys are not wake up source of standby, suspend to RAM and Hibernate states.

-  Activity of the USB device is dependent on the driver support.

### Modem Ring

PCMCIA Modem

The function of waking up the system from Standby is not supported.

 MiniPCI Modem

Activity on the modem ring line will wake the system from Standby if OS enable modem ring wake up.

 USB Modem

The function of waking up the system from Standby is not supported.

### LAN

PCMCIA LAN

The function of waking up the system from Standby is not supported.

 MiniPCI LAN

MiniPCI LAN can wake the system from Standby if OS enable wake up LAN.

 USB LAN

The function of waking up the system from Standby is not supported.

### Real Time Clock Alarm

The Real Time Clock alarm interrupt will wake the system from Standby and Hibernate.

### PC-Card Wake Up

PC-Cards can wake the system up in ACPI mode but cannot wake up the system in APM modes.

### PCI PME# Signal

The PCI PME# signal can wake the system up in ACPI mode but cannot wake up the system in APM modes.

### USB Device

If USB driver is loaded, the driver will wake the system from Standby.

### Critical Low Battery

Critical low battery event can’t wake the system from Standby in ACPI mode.

## Power Management – ACPI

### Introductions

The Advanced Configuration and Power Interface (ACPI) is a well-specified power management and configuration mechanism.  It evolves the existing collection of power management codes, APM, PnP BIOS, and Etc.

### Power State Transition Diagram

The state transition diagram in ACPI mode is as follows:

| From (State) | Leave By Condition | Enter (State) |
| --- | --- | --- |
| S1/S3 | Power Button | S0 |
|  | Modem ring |  |
|  | MiniPCI LAN |  |
|  | Alarm |  |
|  | USB |  |
| S4 | Resume button request |  |
| S0 | Press power button (depends on ACPI OS setting) | S1/S3 |
|  | Press lid switch (depends on ACPI OS setting) |  |
|  | Standby icon in shutdown menu in Windows. |  |
|  | ACPI OS timer expired |  |
|  | Critical low battery (depends on ACPI OS setting) |  |
| S0 | Critical low battery (depends on ACPI OS setting) | S4 |
|  | Press lid switch (depends on ACPI OS setting) |  |
|  | Press power button (depends on ACPI OS setting) |  |

### Embedded controller

The keyboard controller will act as the ACPI embedded controller and support the ACPI EC protocol and interface.

### Storage Devices and Batteries

Possible storage devices are FDD, HDD, CD-ROM and DVD-ROM Configurations

-  Floppy Disk and Hard Disk, CD-ROM and DVD-ROM

The BIOS must report that the correct types of these devices are present even if the drive is not installed in the system during POST.  Two devices, which belong to the same category, are not supported in this notebook.

-  Batteries

The BIOS must follow ACPI specification and report the correct number of the installed battery and status.

### Bootable Device

The system is capable of booting from onboard HDD, FDD, CD ROM and DVD-ROM.

### Docking Features

This product does not support docking station or port replicator.

### PC2001

The notebook must meet Microsoft Logo requirements in accordance with the PC2001 Guide and the Microsoft Logo test programs.

## Miscellaneous Features

### BIOS ROM

The system BIOS and Keyboard BIOS share one single flash ROM.  The size of the flash ROM is 512KB.

### USB Support

The driver provides the USB device support of this notebook after loading the operating system.

### IDE interface

The IDE device supported in the module bay will occupy secondary/master channel.

### Flash utility – one BIOS ROM only

The flash utility can be used to program both system and keyboard BIOS at the same time.

### VGA Support

Support AGP 2.0 that improved the sustained bandwidth that is critical to the enhanced 3D and video performance.

Following describes the expected behavior when a video monitor is connected to the VGA port on the notebook .The feature needs VGA driver support

The BIOS will use both the RGB and pin 11 methods to determine the presence of an external VGA monitor.

### Video modes supported on the secondary display path (need VGA driver support)

Supported video modes and timings please refer to the technical reference of VGA vendor.  In particular, text mode and standard VGA modes are not supported.

### CPU Speed

AC in  cpu runs in full speed

Batty only  cpu runs in lowest speed

### S4 Resume

System can’t boot from any device and “F2” function key beside the HDD.

## Customer Specific Features

### Display of System Type and BIOS Version Number on Boot

〈System Type〉BIOS Version V1.00*

Note:

-  The numbers of BIOS version will be changed.

### Configuration Requirements

| IRQ | Hardware |
| --- | --- |
| 00 | System Timer |
| 01 | Keyboard |
| 02 | Programmable Interrupt Controller |
| 03 | Fast Infrared Port |
| 04 | Free |
| 05 | SD Device |
| 06 | Free |
| 07 | Printer Port (LPT1) |
| 08 | Real Time Clock |
| 09 | SCI IRQ used by ACPI bus , Audio, CardBus, PCI Fast Ethernet , Modem , IEEE 1394, Wireless LAN, USB,USB2.0 and VGA. |
| 10 | Free |
| 11 | Free |
| 12 | PS/2 Mouse |
| 13 | Numeric data processor |
| 14 | Ultra ATA Storage Controller & Primary IDE controller |
| 15 | Ultra ATA Storage Controller & Secondary IDE controller |

| DMA | Hardware |
| --- | --- |
| 01 | Infrared Communications Controller |
| 02 | Standard Floppy Disk Controller |
| 03 | Free |
| 04 | DMA controller |

Notes:

-  For both IRQ and DMA assignment, if the resource is insufficient, the functionality of the conflicted devices will not be well.

### CMOS RAM management

The BIOS will automatically update certain information in CMOS on each boot.  This information includes:

-  DRAM size and configuration

-  Hard disk configuration

-  Always report the existence of one FDD.

If the CMOS RAM fails checksum or a power lost on CMOS battery is detected during boot, an appropriate error message will be displayed:

-  Establishing default CMOS configuration

-  Run SETUP to change configuration

The system BIOS must automatically load default values defined in the setup menu during POST when encounter these problems.  The user must not be required to take any action to continue the rest of POST (or entering SETUP).

### System Management BIOS(SM BIOS) version 2.31

Limited DMI 2.0 BIOS information are provided:

BIOS version number is type 0 data item.

Type 1:

-  System serial number – 64 alphanumeric characters with 12-character  bundle number

-  System manufacturer name – ‘COMPAL’

-  System product name – 32 alphanumeric characters

-  System version – 32 alphanumeric characters

### EEPROM

There is one EEPROM that is used to store many important system and user data in the notebook.  The size of the EEPROM is 2K bytes.

The EEPROM map is listing as below:

| Name | Offset | Comments |
| --- | --- | --- |
| System Serial Number | 00h – 1Fh 20h – 3Fh | 32 bytes for Serial number. 32 bytes for Bundle number. |
| Unused | 40h – 4Fh | Unused. |
| System version | 50h – 6Fh | 32 bytes for System version. |
| UUID | 70h – 7Fh | 16 bytes for UUID. |
| System product name | 80h – 9Fh | 32 bytes for System product name. |
| DMI type 11 | A0h– DDh | 62 bytes for DMI type 11 |
| Unused | DEh– DFh | Unused |
| GUID | E0h – E7h | 8 bytes for GUID |
| Unused | E8h – EFh | Unused |
| Keyboard type | F0h | Define for US/UK/JP keyboard |
| Keyboard BIOS used | F1h | 1 byte for Keyboard BIOS used |
| Branding | F2h | 1 byte for Branding. |
| Reserved for Compal factory | F3h – F4h | Reserved 2 bytes for Compal factory used |
| Reserved for keyboard | F5h – F6h | Reserved 2 bytes for keyboard used |
| Unused | F7h – FDh | Unused |
| EEPROM initialized flag | FEh – FFh | Set to 55AAh when the EEPROM get initialized. |
| Reserved | 200h - 3FFh | Reserved for BIOS Multiboot III function |
| Reserved | 7A0h – 7DFh | 64 bytes for Factory Process |

### “C” Key

During POST, pressing “C” key could boot from CDROM, instead of changing the boot sequence.

### “ESC” Key

During POST, pressing “ESC” key could enable boot menu and exit quick boot logo to see the POST message, instead of changing the boot sequence.

### “F12” Key

During POST, pressing “F12” key could boot from NETWORK, instead of changing the boot sequence.

## System Setup

### Invoking setup

The setup function can only be invoked by pressing F2 when “Press <F2> to enter Setup” message is prompted on the bottom of screen during POST.

The setup uses a menu driven interface to allow the user to configure their system. The features are divided into 5 parts as follows:

 Main	    Allows the user to specify standard IBM PC AT system parameters.

 Advanced  Provides advanced settings of the system.

 Security	   Provides security settings of the system.

 Boot	   Allows the user to specify the boot options.

 Exit	   Allows the user to save CMOS setting and exit Setup.

During setup, all Fn function keys and power saving functions are disabled.

### Setup screens

#### Main Menu

System Memory

This field reports the memory size of system base memory.  The size is fixed to 640KB.

 Extended Memory

Extended Memory size = Total memory size

 CPU Speed

CPU Speed = Max speed

##### System Date and Time

The hour is displayed with 24-hour format. The values set in these two fields take effect immediately.

##### Quiet Boot

 Enabled: Customer Logo is displayed, and Summary Screen is disabled

 Disabled: Customer Logo is not displayed, and Summary Screen is enabled.

##### LCD Auto DIM

 Enabled: Decrease one level of brightness if AC-In is plugged out.

 Disabled: No change of brightness if AC-In is plugged out.

##### Network Boot

 Enabled: Enable PXE boot from LAN.

 Disabled: Disable EXE boot from LAN.

#### Advanced menu

##### FIR Port

Configure the system’s Infrared port using options:

 Disabled:   Disabled.

 Enabled: User configuration – for advanced users only.

Note:

 Depending on your operating system, disabling an unused device may help free system resources for other devices.

COM B I/O Settings

 Set the base I/O address and IRQ for Infrared port.

 Disabled/COM1,3F8,IRQ4/COM2,2F8,IRQ3/COM3,3E8,IRQ4/COM4,2E8,IRQ3

DMA Setting for Fast IR

 DMA 1/DMA 2/DMA 3

Mode Setting

 Normal(16550)/ IrDA(HPSI

R)/ ASK IR/ FAST IR

##### LPT Port

Configure parallel port using options:

 Disabled: Disabled.

 Enabled: User configuration – for advanced users only.

Note:

 Depending on your operating system, disabling an unused device may help free system resources for other devices.

Port Definition

Set the mode for the parallel port:

 Standard AT: Normal mode (AT compatible).

 Bi-directional: Bi-directional mode (PS/2 compatible).

 Enhanced Parallel(EPP): EPP mode.

 Extended Capabilities(ECP): ECP mode (requires DMA channel).

Port Address

 Set the base I/O address for the parallel port.  When Mode is selected as EPP mode, ‘3BC’ will not be available.

 None / LPT1, 378, IRQ 7 / LPT2, 278, IRQ5 / LPT3 ,3BC, IRQ7

DMA Setting For ECP Mode

If select ECP mode then DMA default DMA 1.

##### Legacy USB Support

 Disabled: Disable support for Legacy Universal Serial Bus.

 Enabled: Enable support for Legacy Universal Serial Bus.

#### Security Menu

The following is Security menu if both of passwords are disabled, or enter Supervisor password when password is enabled:

##### Set Supervisor/ User Password

Enter	this field always shows the message.

If password on boot is required, the password must be set otherwise it cannot be enabled.

The formats of the password are as follows:

Length		10 characters.

Characters	Alphanumeric keys only. The shift status i.e. Ctrl, Shift, Alt, and Capital are ignored.

Password on Boot

Allows the user to specify whether or not a password is required to boot.

 Enabled: a password is required to start the system.

 Disabled: Do not apply password to the system.

#### Boot Menu

This menu allows the user to decide the order of boot devices to load the operating system.  Bootable devices include the diskette drive in module bay, the onboard hard disk drive and the CD-ROM in module bay.

#### Exit Menu

##### Exit Saving Changes

Allows the user to save changes to CMOS and reboot system. The following message is prompted when user press “Enter” on the item.

 Yes: Exit SETUP and reboot

 No: Back to previous screen

##### Exit Discarding Changes

##### Load Setup Defaults

Allows the user loads default value in CMOS Setup. The following message is prompted when user press “Enter” on this item:

It still stay in Setup when press a key.

##### Discard Changes

Allows the user loads previous value in CMOS Setup. The following message is prompted when press “Enter” on this item:

It still stay in Setup when press a key.

## OS Compatibility

Windows XP Home Edition /Professional.

## Software Specification for KB-BIOS

### General purpose

This document define the EC specification with standard interface and also define the special feature for OEM function .It’s useful for software engineer  to access EC status, and for SA test guide.

### Features

-  Advanced Power Management 1.2 support

-  ACPI1.0 b and PC2001 compliant

-  Support SMBus specification V1.0

-  Hot keys for system control

-  External LED control

-  Battery scope report and control

-  Sticky key support

-  Power switch control

-  Speaker control

-  Port replicate

-  Extra key emulation

-  Two host interface channels support

-  Internal Keyboard country selection

### Types of KB-BIOS provided

-  Standard version

-  Support for US(87)/UK(88)/Japanese(90) keyboard.

-  KB-BIOS command support with SYSTEM-BIOS

### Command set 40h-4Fh for OEM defined through Port60/64 and Port62/66

Command Set (from system's point of view) via 60/64 and 62/66

| CMD | DATA | Description | return |
| --- | --- | --- | --- |
| 40h |  | Boot fail restart |  |
|  | 0x01-0x7F | Boot fail restart, write in a byte to EC and enable the timer. This command called by BIOS and will cause the system reboot after the byte count down to zero if system still no reset the counter. It could make sure the system success boot up. | None |
| 41h |  | EC state notification |  |
|  | A0h | Return core code version number | One byte |
|  | A1h | Return platform id | ‘COMPAL’ |
| 42h |  | Bank assign for EEPROM | None |
|  | 0x00-0x07 | Bank assign for EEPROM,work with 4D/4E command replace 4B/4C command | None |
| 43h | 00h - FFh | Get RAM value from EC, Host can use this command to send address to get OEM RAM value. The address range are from 00h to DFh. | 1 WORD |
| 44h | None | Get project ID | 0x0A |
| 45h |  | Hook for every projects |  |
|  | 70h | Get battery cycle count. | 1 Byte |
|  | 71h | Get battery learning status. | 1 Byte |
|  | 72h | LLB resume mode disable. | None |
|  | 73h | LLB resume mode enable. | None |
|  | 74h | Disable switch LCD/TV/CRT mode. | None |
|  | 75h | Enable switch LCD/TV/CRT mode. | None |
|  | 76h | Switch LCD/TV/CRT mode by driver. | None |
|  | 80h | Enable touch pad. | None |
|  | 81h | Disable touch pad. | None |
|  | 82h | Enable touch pad enable/disable function. | None |
|  | 83h | Disable touch pad enable/disable function. | None |
| 46h |  | fan speed read |  |
|  | 01h |  |  |
|  | 02h | Reading FAN speed from FAN1 and FAN2 | None |
|  | 03h | Fan RPM control by EC | None |
|  | default | Fan RPM value,and Fan RPM control by OS | None |
| 47h |  | Speaker mute On/Off,LED control. |  |
|  | 80h | Return back the control right to EC | None |
|  | 81h | System Led on and control by OS | None |
|  | 82h | Power management Led on and control by OS | None |
|  | 83h | Charge Led on and control by OS | None |
|  | 84h | Discharge LED on and control by OS | None |
|  | 85h | Direct CD Led on and control by OS (not support) | None |
|  | 86h | Blue Tooth Led on and control by OS (not support) | None |
|  | 87h | One touch button Led on and control by OS (not support) | None |
|  | 88h | All LED off and control by OS | None |
|  | 89h | Hdd LED on and control by OS | None |
|  | 8Ah | Fdd LED on and control by OS | None |
|  | 8Bh | MP3 LED on and control by OS | None |
|  | 8Ch | 2ND battery LED on and control by OS | None |
|  | A7h | Mute off(not support) | None |
|  | A8h | Mute on(not support) | None |
| 48h |  | Brightness control |  |
|  | 0h | Time out to turn off panel counter | None |
|  | 1h | Adjuest brightness duty cycle | None |
| 49h |  | Thermal control notification |  |
|  | A0h | FAN off and throttling disable | None |
|  | A1h | FAN speed level 1 on and throttling disable | None |
|  | A2h | FAN speed level 2 on and throttling disable | None |
|  | A3h | throttling enable | None |
|  | A4h | FAN speed level 3 on and throttling disable | None |
| 49h | A5h | FAN speed level 3 on and throttling disable | None |
|  |  | If FAN_NUM >= 2 |  |
|  | B0h | FAN off | None |
|  | B1h | FAN on speed1 | None |
|  | B2h | FAN on speed2 | None |
|  | B3h | Throttling enable | None |
|  | B4h | FAN on speed3 | None |
|  | B5h | FAN on speed4 | None |
|  |  | If FAN_NUM >= 3 | None |
|  | C0h | FAN off | None |
|  | C1h | FAN on speed1 | None |
|  | C2h | FAN on speed2 | None |
|  | C3h | Throttling enable | None |
|  | C4h | FAN on speed3 | None |
|  | C5h | FAN on speed4 | None |
| 4Ah |  | Auto into S2R(Delay about 4 Secs) or S2D and resume by timeout, This command provided engineer to verify S2R or S2D and resume function is OK or not Resume count(second base ), Range is from 0x02 to 0x7F Bit7 = 0  Enabled S2R function  Enabled S2D function |  |
|  | 0x02-0x7F | Enabled S2R function | None |
|  | 0x80-0xFF | Enabled S2D function | None |
| 4Bh |  | Write the data to device through SMBus interface |  |
|  | 0. 00h-FFh | Slave address of device | Byte |
|  | 1. 00h-FFh | Lo byte address if device is EEPROM otherwise is command or register | Byte |
|  | 2. 00h-FFh | Hi byte address if device is EEPROM, otherwise is zero | Byte |
|  | 3. 00h-FFh | data byte to write | 0=Write OK 0xFE = Fail |
| 4Ch |  | Read data from devices through SMBus |  |
|  | 0. 00h-FFh | Slave address of device | Byte |
|  | 1. 00h-FFh | Lo byte address if device is EEPROM otherwise is command or register | Byte |
|  | 2. 00h-FFh | Hi byte address if device is EEPROM, otherwise is zero | data byte (00h-FFh) |
| 4Dh |  | Write byte into EEPROM |  |
|  | 1. 00h-FFh | EEPROM address |  |
|  | 2. 00h-FFh | Data byte for write byte | 0=Write OK 0xFE = Fail |
| 4Eh |  | Read byte from EEPROM |  |
|  | 00h-FFh | EEPROM address | 0=Write OK 0xFE = Fail |
| 4Fh |  | Reserved for R591 utility | None |

### Command set 50h-5Fh for OEM defined through Port60/64 and        Port62/66

Command Set (from system's point of view) via 60/64 and 62/66

| CMD | DATA | Description | return |
| --- | --- | --- | --- |
| 50h | None | Get Docking status. | A0h = No dock A6h = simple docked |
| 51h | None | Get revision number of KB-BIOS | BIOS Rev. 3 bytes byte0 : bit0-bit2 = major number(0-7) bit3-bit7 = type of KB-BIOS e.g. 0 = A, 1=B and so on.. byte1 minor revision number(0-9) byte2 If it is 00h then system display “ROM”,other It is “T01” if it is 01h and so on.. . |
| 52h | None | Hook for every projects(Get platform ID) | 5 Bytes “BCL50” |
| 53h | None | Reserved | None |
| 54h | 0x00-0xFF | EC CMOS RAM read | Data byte from CMOS |
| 55h |  | EC CMOS RAM write |  |
|  | 0x00-0xFF | CMOS address offset |  |
|  | 0x00-0xFF | data byte | 0x00 => pass 0x01 => fail |
| 56h |  | Get SMI trigger source | One byte |
|  |  | No event | 80h |
|  |  | Ask suspend(On mode) | A0h |
|  |  | brightness level update | A1h |
|  |  | contrast level update | A2h |
|  |  | audio volume decreased | A3h |
|  |  | audio volume increased | A4h |
|  |  | Lid open | A5h |
|  |  | Lid closed | A6h |
|  |  | External device plugged | A7h |
|  |  | External device removed | A8h |
|  |  | Bluetooth wake up event | A9h |
|  |  | Bluetooth switch event | Aah |
|  |  | Scr expand event | Abh |
|  |  | Display swap event | Ach |
| 56h |  | Cpu fast event | Adh |
|  |  | Cpu slow event | Aeh |
|  |  | Pop up event | Afh |
|  |  | Resume request from suspend | B0h |
|  |  | Ask time out event | B1h |
|  |  | Battery life in critical low state | B2h |
|  |  | Battery life in low power state | B3h |
|  |  | Standby request | B4h |
|  |  | Battery Plug-In | B5h |
|  |  | Battery Plug-Out | B6h |
|  |  | Reserved | B7h |
|  |  | Suspend to RAM request | B8h |
|  |  | Save to DISK request | B9h |
|  |  | Docked request | Bah |
|  |  | Undock request | Bbh |
|  |  | Reserved | Bch |
|  |  | Thermal change event | Bdh |
|  |  | Write LM75 event | Beh |
|  |  | SMBus event | Bfh |
|  |  | Password event | C0h |
|  |  | mute function toggle | C1h |
|  |  | Power button pressed | C2h |
|  |  | TV out toggle | C3h |
|  |  | Beep Alarm event | C4h |
|  |  | Reserved | C5h |
|  |  | Change use battery | C6h |
|  |  | AC power plug-in | C7h |
|  |  | AC power plug-out | C8h |
|  |  | IR toggle event | C9h |
|  |  | Modem Ring In | Cah |
|  |  | Unload OS Ultra Base Devices | Cbh |
|  |  | Surprise undock event | Cch |
|  |  | Battery polling | Cdh |
|  |  | PME signal active | Ceh |
|  |  | Mouse hot plug event | Cfh |
| 56h |  | CRT plug in/out event | D0h |
|  |  | Sleep button event | D1h |
|  |  | RTC date/time update event | D2h |
|  |  | LCD rotate event | D5h |
|  |  | Device change event | F0h |
|  |  | Bluetooth lan event | F1h |
|  |  | no event | FFh |
| 57h | None | Module identification | One byte bit0 : Main HDD exist bit1 : Int. FDD exist bit2 : CD_ROM exist bit3 : 2nd HDD exist bit4 : LS120 exist bit5 : Ext. FDD bit6 : CRT plug In/Out(1=in) other bit : Reserved |
| 58h | 0x00-0xFF | Set flat panel type | None |
| 59h |  | System state notification | None |
|  | 70h | Sticky key mode enable | None |
|  | 71h | Sticky key mode disable | None |
|  | 72h | WirelessOn | None |
|  | 73h | WirelessOff | None |
|  | 74h | Change command to command48 | None |
|  | 75h | Panel BRIGHTNESS adjust | None |
|  | 76h | Fan full on disable | None |
|  | 77h | Fan full on enable | None |
|  | 78h | Fan Fine Tune Disable | None |
|  | 79h | Fan Fine Tune Enable | None |
|  | 80h | start get LCD status panel information from EEPROM | None |
|  | 90h | One touch button application allow to send scan code(user button) if user pressed | None |
|  | 91h | One touch button application don’t allow to send scan code(user button) if user pressed | None |
|  | 92h | Mail message is waiting(no support in Hurricane) | None |
|  | 93h | Mail message end of waiting(no support in Hurricane) | None |
|  | 94h | Mute on | None |
|  | 95h | Mute off | None |
|  | 9ah | Ac off(cut off AC power) | None |
|  | 9bh | Ac on | None |
|  | 9ch | Enable lid switch resume function | None |
|  | 9dh | Disable lid switch resume function | None |
|  | A2h | System enter S2D(S4) state | None |
|  | A3h | System enter beep mode for battery LB state in CMOS setup | None |
|  | A4h | System enter quiet mode for battery LB state in CMOS setup | None |
|  | A5h | Fan control by EC | None |
|  | A8h | Fan control by OS | None |
|  | A9h | external PS2 only | None |
|  | Aah | Both enable external PS2 and internal touch pad | None |
|  | Ach | Auto enable/disable external PS2 and internal touch pad | None |
|  | B1h | System into standby | None |
|  | B2h | Resume from standby | None |
|  | B5h | VGA suspend enable | None |
|  | B6h | VGA suspend disable | None |
|  | B7h | Modem ring enable | None |
|  | B8h | Modem ring disable | None |
|  | B9h | PME enable | None |
|  | Bah | PME disable | None |
|  | Bbh | In S4 status | None |
|  | Bch | Resume from S4 status | None |
|  | C1h | force battery pack auto learning | None |
|  | C2h | disable battery pack learning | None |
|  | C3h | SMI/SCI Trigger event enable | None |
|  | C4h | SMI/SCI Trigger event disable | None |
|  | Cbh | PCMCIA suspend disable | None |
|  | Cch | PCMCIA suspend enable | None |
|  | Cdh | Wake up LAN disable | None |
|  | Ceh | Wake up LAN enable | None |
|  | D0h | Disable IRQ1 | None |
|  | D1h | Enable IRQ1 | None |
|  | D2h | Beep alarm 100mS | None |
|  | D4h | Resume OK | None |
|  | D5h | PCMCIA reset off | None |
|  | D6h | PCMCIA reset on | None |
|  | D7h | Battery stop charge enable | None |
|  | D8h | Battery stop charge disable | None |
|  | E1h | Turn LCD back-light on | None |
|  | E2h | Turn LCD back-light off | None |
|  | E3h | Enable Fn emultion of Ctrl+Alt external KBD | None |
|  | E4h | Disable Fn emultion of Ctrl+Alt external KBD | None |
|  | E5h | Select US keyboard Matrix | None |
|  | E6h | Select JP keyboard Matrix | None |
|  | E7h | Select UK keyboard Matrix | None |
|  | E8h | EC into ACPI mode | None |
|  | E9h | Non-ACPI mode (EC default) | None |
|  | Eah | Enable & Initial USART function | None |
|  | F2h | disable watchdog | None |
|  | F3h | enable watchdog | None |
|  | F4h | enable RTC access by EC | None |
|  | F5h | disable RTC access by EC | None |
|  | F6h | Clear header of Boot code | None |
|  | F7h | Restart system and Clear header of Boot code | None |
|  | F8h | Shut down system and Clear header of Boot code | None |
|  | F9h | Clear header of Boot code | None |
| 5Ah |  | RTC update |  |
| 1 | A0h | Update Year of RTC ,Year(00-99) BCD format |  |
| 2 | 00-99 | Year which want to display | None |
| 1 | A1h | Update Month of RTC ,Month ( 1..12) BCD format |  |
| 2 | 01-12 | Month which want to display | None |
| 1 | A2h | Update DAY of RTC ,Day(01-07) BCD format |  |
| 2 | 01-07 | Day which want to display | None |
| 1 | A3h | Update HOUR of RTC ,Hour(00-23) BCD format |  |
| 2 | 00-23 | Hour which want to display | None |
| 1 | A4h | Update Minute of RTC ,Minute (0..59), BCD format |  |
| 2 | 00-59 | Minutes which want to display | None |
| 1 | A5h | Update Second of RTC ,Second (0..59), BCD format |  |
| 2 | 00-59 | Seconds which want to display | None |
| 5Bh |  | Reserved |  |
| 5Ch | None | Get brightness level | current brightness level (0x00-0x0a) |
| 5Dh |  | Set brightness level |  |
|  | 0x00h-0x0ah | new brightness level | None |
| 5Eh | None | Get contrast level | Current contrast level (0x00-0x40) |
| 5Fh |  | Set contrast level |  |
|  | 0x00h-0x40h | new contrast level | None |

### Hot keys for system control

Definitions

All Fn Key will support Sticky key mode.

| Fn+ESC | Enter Suspend mode |
| --- | --- |
| Fn+↓ | Decrease brightness (total 10 levels) |
| Fn+ ↑ | Increase brightness (total 10 levels) |
| Fn+F5 | Power on display When pressing the hot key, the display device will switch among CRT only, LCD only and simultaneous display. It will not update the setting of option ‘Power on Display’ in system setup. |
| Fn+PgUp | Audio volume up |
| Fn+PgDn | Audio volume down |
| Fn+F10 | Pad Lock Pressing this hot key can enable/disable the embedded keypad. In this mode the keypad is cursor function. |
| Fn+F11 | Num Lock Pressing this hot key can enable/disable the embedded numeric keypad. |
| Fn+F12 | Scroll Lock |
| Fn+END | Mute toggle (no beep from disable to enable) |

-  After rebooting, pad lock is set to off and Num lock is also too.  In this state, the embedded cursor/number pad is not enabled on the notebook keyboard.

-  When the embedded keypad is on, holding down Fn will turn the embedded keypad off.

Note:

-  Hot keys for brightness/Volume up /dn adjustment are in repeat mode,

- others will only be updated once for each key depression.

### Audio volume output control

-  Use hot keys Fn+PgUp/Fn+PgDn  for Increasing/Decreasing respectively, it controls  the volume output of the audio chip.

-  Use hot key Fn+End to enable/disable audio/speaker output. The default state  enabled while system is turned on

### External Buttons status report and control

Define the function of buttons which is controlled by EC.

### Power Switch

-  If system is Off/S2D : System will be turned on while Power switch is depressed

- by more than 500 ms with or without  AC insert

-  If system is in S2R/Standby state : System will resume while Power switch is depressed by more than 100 ms

### 802.11b & bluetooth power button

-  Turn on 802.11b RF power and Bluetooth power. ( debounce 100 ms )

### User Buttons

-  Support 2 user buttons,it will launch homologous application.

### External LEDs status report and control

Define the Led  display status.

### Definitions of Lock LEDs

-  Num Lock LED: Num Lock  State of  Keyboard

-  Pad Lock LED: Pad Lock State of keyboard

-  Caps Lock LED: Caps Lock State of keyboard

### Definitions of System state LED

-  Blinking: System stand-by/suspend (Amber on for 1 second every 2 seconds)

-  Blue Color solid on: System on.

-  Blue color off: System off .

### Definitions of DC-DC state LED

-  There is one dual-color LED indicator both of Blue and Amber color.

-  LED colors and definitations :

Blue color and Yellow color defined for battery charging and discharging state. The definitions are listed below:

| Battery State | LED colors | Definitation |
| --- | --- | --- |
| Charging | Blue and blinking | Battery charging with AC . (LED on 1 second every 2 seconds) |
|  | Blue and solid on | Battery full by AC charge |
|  | Blue & Amber LED off | Battery abnormal stop chargeing with AC (BadCell/OverTemp) |
| Discharging | Amber and blinking | Battery within low state (10%)LED on lsecomd every 4 second |
|  | Amber and solid on | Battery not in low or critical low state. It in discharging state. |
|  | Amber and blinking | Battery within critical low state(<=3%) LED on lsecond Every 4 second |

### Definitions of IDE accessing state LED

-  Reflect the activities of HDD.

(If CD-ROM activing, HW will send singal to EC then EC control the LED )

### Definitions of CD-ROM state LED

-  If CD-ROM module exist, the LED reflect the activities of CD-ROM.

(If CD-ROM activing, HW will send singal to EC then EC control the LED )

### Definitions of Bluetooth/802.11b on/off State LED

-  Blue color: Bluetooth/Wireless  power on.

-  Off: Bluetooth/Wireless  power off.

### Battery status report and control

Define the battery type spec and battery protection function.

### Battery status

-  There are four battery states for each battery pack depend on getting the battery state through SMBus protocol from Smart battery pack: full, normal, low, critical low.

-  The battery gas-gauge and level of low power states should base on ‘current’ system configuration.

-  Battery turn on system condition : gasgauge > 5%

### Battery discharge/charging control

| Charging | Dischargung | Action |
| --- | --- | --- |
| 0℃<T<50℃ T>60℃ |  | Stop charging & Battery Led off |
| T>73℃ | T>73℃ | Shut down |
|  | R.S.O.C. ≦10% | LB(Beeping) |
|  | R.S.O.C. < 3% | LLB Dependent on OS |
|  | Battery Voltage ≦ 12.0V. | LLC Shut down System |
|  | R.S.O.C < 5% during system is in S2R mode. | S2D |
| Fast Charge Time out: 8 Hours Trickle Charge Time Out: 1 Hour. |  | Battery BAD & Battery Led off |
| Voltage≧17.4V |  | OverVoltage & Battery Led off |

### In ACPI mode

-  System should 'Save to Disk'(S2D) or beeping(Low condition) depend on OS setting .

### Battery type

-  The KB-BIOS will support for smart battery pack by SMBus protocol.

-  ACPI1.0b and PC2001 Compliant,with PC2001 spec “A mobile system must use

- a Smart Battery or an ACPI control method battery”,our currently design is ACPI control method battery.

-  Compatible with Intel’s SMBus and Philip’s I2C bus protocol.

### KB-BIOS Power management support

EC will support S1(sleep mode),S3(standby mode),S4(suspend to disk) mode to save the power comsumption.

### Power states

-  Sleep mode

-  LCD panel back-light off

-  Save to RAM

-  keyboard(int./ext.) scanning off

-  Save to DISK

-  no actions except turning off system with AC exist or turn off KBC without AC.

### SMI/SCI/SWI/SBS/SPB events(To be Changed)

-  Following list is which events(SMI/SCI/SWI/SBS) will be generated under different OS.

|  | APM Mode |  |  |  | ACPI mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Brightness level changed | SMI | - | - | A1h | SCI | - | - | - | 11h | - |
| Contrast level changed | SMI | - | - | A2h | SMI | - | - | - | 12h | - |
| CoverLid close | SMI | - | - | A6h | SCI | - | - | - | 16h | - |
| Display toggle | SMI | - | - | Ach | SCI | - | - | - | 1ch | - |
| Battery in critical low | SMI | SMI | - | B2h | SCI | SWI | - | - | 22h | - |
| Battery in low state | SMI | - | - | B3h | SCI | - | - | - | 23h | - |
| Standby request | SMI | - | - | B4h | - | - | - | - | 24h | - |
| Battery pack plugin | SMI | - | - | B5h | SBS | - | - | - | 25h | - |
| Battery pack removed | SMI | - | - | B6h | SBS | - | - | - | 25h | - |
| Suspend To RAM request | SMI | - | - | B8h | SCI | - | - | - | 28h | - |
| Save To DISK request | SMI | - | - | B9h | - | - | - | - | 29h | - |
| Docking in | SMI | - | - | Bah | SCI | SWI | - | - | 2ah | - |
| Undock | SMI | - | - | BBh | SCI | - | - | - | 2bh | - |
| AC plugin | SMI | - | - | C7h | SBS | - | - | - | 37h | - |
| AC removed | SMI | - | - | C8h | SBS | - | - | - | 38h | - |
| Modem ringin | SMI | - | - | Cah | - | SWI | - | - | 3ah | 04 |
| PME signal active | SMI | SMI | - | Ceh | SCI | SWI | - | - | 3eh | 02 |
| CRT plugin/out | SMI |  |  | D0h | SCI |  |  |  | 40h |  |
| RTC Update | SMI |  |  | D2h | SCI |  |  |  | 42h |  |

### Thermal Status Report and Fan Control

EC will control fan on/off function according to the CPU temperature(EC can get temperature from thermal sensor through SMBus) .In currently spec,Fan will be off when temperature below 55℃ ,and if temperature over 95℃ five times,EC will auto turn off system to protect CPU.For detail data please reference follows table.

| Fan State & System State | Temperature |
| --- | --- |
| Fan Off | 55℃ |
| Fan1=4X(T-60)+95 | τ |
| Fan2=8X(T-70)+135 | τ |
| Throttling off | 85℃ |
| Throttling on | 90℃ |
| Turn off Fan & shut down | 95℃ |

Extra keyboard emulation

### Windows key emulation

-  Left/Right windows keys both depressed: Fn and Application key depressed simultaneously.

-  Right window key : press Fn+Left window key.

### Fn Key emulation (Not support)

-  External keyboard Fn-key emulation except Pad-Lock by pressing both Left Ctrl

- and Left Alt of external keyboard. It will not work on USB keyboard.

### Internal Key-Pad mode control

-  Numeric key: System NumLocck state.

- (internal key-pad will disable when external keyboard  is exist)

Two host interface channels support

### Keyboard and mouse interface transfer port

-  One channel is dedicated for the keyboard and mouse data transfer(host address 60h and 64h). The Keyboard and Mouse channel of KBC is compatible to the legacy 8042 host interface. It is base on two registers: Command/Data and Status

-  The KB-BIOS interrupt generates IRQ1(Keyboard) and IRQ12(Mouse) for system.

### Power management  interface transfer port

-  The other for the power management function(host address 62h and 66h). The

- Power Management channel of KBC structure and operation are similar to those of the Keyboard/Mouse channel.

### Support three independent devices

-  The KBC provides three data transfer channels. Each channel has two quasi-bidirectional signals that are used for the direct interface to an external keyboard, mouse or any other PS/2 compatible pointing device.

-  The three channels are identical and thus allow the connector ports to be interchangeable.

Devices PnP configuration

### Hot Plug-and-Play support

-  The KBC watches both external devices, checking if the devices have recently

- been plugged in or unplugged. The Hot pluggability of external PS2 devices feature detects the attachment or removal of these devices.

### Hot swapping control

-  When the device is plugged in, the software automatically initializes the state of

- that device, checks port swapping, and setup the KBC to handle dual-device operation. In dual-device operation, the internal device is set in the same state as external device. When the external device is unplugged, the internal device becomes the primary device.

ACPI EC interface Specification support

### ACPI interface support

-  The KBC provides support for Advance Configuration and Power Interface specification(ACPI) Embedded Controller interface.

### EC command support

-  The 2nd (Power Management) host interface channel of the KBC is dedicated to

- this function.

-  All EC commands defined in the ACPI specification - Read/Write, Burst Mode enable/disable and Query command - are supported.

Internal keyboard change Configuration

### US/UK/JP country option

-  The KBC supports three country selection by KBD_SEL application.

-  You can key in KBD_SEL get the syntax for your option at DOS prompt.

### Sticky key support

-  Press shift key 5 times will enable sticky key function.Turns on StickyKeys,which allows you to press a modifier key(CTRL,ALT,or SHIFT),or the windows logo key, and have it remain active until the next time you press a key other then CTRL,ALT,SHIFT,or windows logo key. This is useful for people who have diffculity pressing two keys simultaneously.

EC name space Configuration

### SMBus EC interface ACPI RAM definition

| Offset | Description | Offset | Description |
| --- | --- | --- | --- |
| 60h | SMBus protocol | 84h | SMBus BCNT |
| 61h | SMBus statue Bit0-Bit4 – Status Bit5 - Reserved Bit6 - ALARM Bit7 - DONE | 85h | SMBus alarm address |
| 62h | SMBus Address | 86h | SMBus alarm data 0 |
| 63h | SMBus Command | 87h | SMBus alarm data 1 |
| 64h – 83h | SMBus Data |  |  |

### Word registers to Emulate smart charge RAM definition

| Offset | Description |
| --- | --- |
| 90h – 91h | CHG_MODE0 CHG_MODE1 Bit0 – INHIBIT_CHARGE(0=enabled, 1=inhibit) Bit1 – ENABLE_POLLING(0=disable, 1=enable) Bit2 – POR_RESET(0=Mode unchanged, 1=set power on defaults) Bit3 – RESET_TO_ZERO (0=No change, 1=set charging values to zero) Bit4 – 15 – Reserved |
| 92h – 93h | CHG_STAT0 CHG_STAT1 Bit0 – CHARGE_INHIBITED(Status of bit in CHG_MODE register) Bit1 – MASTER_MODE(Set if HOST controlled & ENABLE_POLL) Bit2 – VOLTAGE_NOTREG(Set if CHG_VOLT not in regulation ) Bit3 – CURRENT_NOTREG (Set if CHG_CURRENT not in regulation) Bit4 – LEVEL_2(Set always at least level 2) Bit5 – LEVEL_3(Set always if level 3 capable) Bit6 – CURRENT_OR(Set if CHG_CURRENT out of range) Bit7 – VOLTAGE_OR(Set if CHG_VOLT out of range) Bit8 – THERMISTOR_OR(Set if thermistor R>100K Ohms)(Open) Bit9 – THERMISTOR_COLD(Set if thermistor R>30K Ohms) (Cold Batt) Bit10 – THERMISTOR_HOT(Set if thermistor R<3K Ohms) (Hot Batt) Bit11 – THERMISTOR_UR(Set if thermistor R<500 Ohms) (Under range) Bit12 – ALARM_INHIBITED(Set if charging inhibited from Alarm) Bit13 – POWER_FAIL(Set if power fail) Bit14 – BATTERY_PRESENT(Set if battery present) Bit15 – AC_PRESENT(Set if charging power source available) |
| 94h –95h | CHG_CURRENT0 CHG_CURRENT1 Bit0-Bit15 – Requested charging current(mA) 0=Turn off charger 65535=Provide maximum safe charger current |
| 96h – 97h | CHG_VOLT0 CHG_VOLT1 Bit0-Bit15 – Requested charging voltage(mV) 0=Turn off charger 65535=Provide maximum safe charger voltage |
| 98h – 99h | CHG_ALARM0 CHG_ALARM1 *** Alarm Bits *** 0x8000 – OVER_CHARGED_ALARM 0x4000 – TERMINATE_CHARGE_ALARM 0x2000 – RESERVED 0x1000 – OVER_TEMP_ALARM 0x0800 – TERMINATE_DISCHARGE_ALARM 0x0400 – RESERVED 0x0200 – REMAINING_CAPACITY_ALARM 0x0100 – REMAINING_TIME_ALARM *** Status Bits *** 0x0080 – INITIALIZED 0x0040 – DISCHARGING 0x0020 – FULLY_CHARGED 0x0010 – FULLY_DISCHARGED *** Error Code *** 0x0000 – 0x000F – All bits set hi prior to AlarmWarning() xmit |

### Word registers to Emulate smart selector RAM definition

| Offset | Description |
| --- | --- |
| 9Ah | SEL_STATE0 Bit0 – PRESENT_A(Set if 1st battery present) Bit1 – PRESENT_B(Set if 2nd battery present) Bit2 – PRESENT_C(Set if 3rd battery present) Bit3 – PRESENT_D(Set if 4th battery present) Bit4 – CHARGE_A(Set if 1st battery be charging) Bit5 – CHARGE_B(Set if 2nd battery be charging) Bit6 – CHARGE_C(Set if 3rd battery be charging) Bit7 – CHARGE_D(Set if 4th battery be charging) |
| 9Bh | SEL_STATE1 Bit0 – PWR_BY_A(Set if system power up by 1st ) Bit1 – PWR_BY_B(Set if system power up by 2nd ) Bit2 – PWR_BY_C(Set if system power up by 3rd ) Bit3 – PWR_BY_D(Set if system power up by 4th ) Bit4 – SMB_A(Set if 1st battery on SMBus) Bit5 – SMB_B(Set if 2nd battery on SMBus) Bit6 – SMB_C(Set if 3rd battery on SMBus) Bit7 – SMB_D(Set if 4th battery on SMBus) |

### EC interface OEM common RAM definition

| Offset | Description |
| --- | --- |
| 9Ch | ACPI_FLAG0 Bit0 –Main HDD (1:exist) Bit1 – Internal FDD (1:exist) Bit2 – Internal CD_COM (1:exist) Bit3 – 2ND HDD (1:exist) Bit4 – LS-120 (1:exist) Bit5 –External FDD(1:exist) Bit6 – CRT-PLUG (1:in) |
| 9Dh | ACPI_FLAG1 Bit0 – Sleep button(1:pressed) Bit1 – Video out button(1:pressed) Bit2 – Decrease Volume(1:pressed) Bit3 – Increase Volume(1:pressed) Bit4 – Mute button(1:pressed) Bit5 – Contrast button(1:pressed) Bit6 – Brightness button(1:pressed) Bit7 – Save to disk button(1:pressed) |
| 9Eh | ACPI_FLAG2 Bit0 – ACPI entry S4 state Bit1 = password enable (Fn+F1), set 1 Bit2=beep alarm enable (Fn+F4), set 1 Bit3= touch pad button(1:enable) Bit4=Fn state (1: fn key down) Bit5= CD/DVD mode selected,set 1 Bit6= Digitial mode selected,set 1 Bit7= CD Lock mode enable,set 1 |
| 9Fh | Reserved |
| A0h | UbStatus: Ultra Base control pin status Bit0 – DPWR, Turn on Dock PCI power(0=off, 1=on) Bit1 – UDRF, Undock request(0=inactive, 1=undock & flash LED) Bit2 – UDRS, Undock request(0=inactive, 1=undock & solid LED) Bit3 – EQBF, Enable Q-Buff(0=disable, 1=enable) Bit4 – DWELL, Docked well LED(0=LED off, 1=LED on) Bit5 – QVCCOK, Dock power ready status(0=No, 1=Yes) Bit7 –CheckEject , (SoftEject request : 0=No 1=Yes ) |
| A1h | DCID: Customer ID Bit0 –DockType0, Dock on or not(0=off, 1=on) Bit1 –DockType1, reserved Bit4 – OS_undock OK Bit5 – OS dock OK Bit6 – Safe Undock OK Bit7 –DockChange, Ultra Base had changed from docked to undock or undock to dock (0=no, 1=yes) |
| A2h | Battery Learning steps. |
| A3h | SYS_STATUS: System indicator Bit0 – S0LED, S0 state LED(0=LED off, 1=LED on) Bit1 – S3LED, S3 state LED(0=LED off, 1=LED on) Bit2 – VGAQ, VGA H/W suspend(0=VGA on, 1=VGA suspend) Bit3 – PCMQ, PCMCIA H/W suspend(0/1=PCMCIA on/suspend ) Bit4 – PCMR, PCMCIA H/W reset (0=disable, 1=enable) Bit5 –ADP,Ac adapter (0=offline, 1=online) Bit6 –SYSR6(reserved) Bit7 –SYSR7(reserved) |
| A4h | WAKEUP_ENABLE: Enable wake up function Bit0 –PMEWAKE(PME Wk Enable:0=Disable, 1=Enable) Bit1 –MDMWAKE (Modem Wk Enable:0=Disable, 1=Enable) Bit2 - LANWAKE(LAN wakeup enable:0=Disable, 1=Enable) Bit3-Bit7 - reserved |
| A5h | Reserved |
| A6h | Reserved |
| A7h | Reserved |
| A8h | Reserved |
| A9h | Reserved |
| AAh | Reserved |
| ABh | Reserved |
| ACh | Shutdown temperature |
| ADh | Temperature index |
| AEh | Bit 0~6 :Throttling level 0 = throttling 12.5% 1 = throttling 25% 8 =throttling 100% Bit7—Sytem throttling HW enable 50% |
| AFh | THERMAL_STATUS Bit0 – MODE (0=Local mode, 1=Remote mode) Bit1 – FANSPDB0(Fan on/off parameter0) Bit2 – FANSPDB1(Fan on/off parameter1) Bit 2 1 ( When control by OS ) 0 0 : Fan off 0 1 : Fan on speed 1 1 0 : Fan on speed 2 1 1 : Fan on speed 3 Bit3 – INITOK ( 0:Control by OS 1:Control by EC ) Bit4 – Fan1 Active Bit5 – Fan2 Active Bit6 – Fan speed timer init OK Bit7 –SKINMODE 0=skin address 90 1=skin address 92 |
| B0h | CPU_TEMP: CPU current temperature |
| B1h | SWI_Events: SWI Event indicators Bit1 – Lid Open, Lid open event (0= off, 1= on) Bit2 – PME, PME event (0= off, 1= on) Bit3 –Power Button, Power button event (0= off, 1= on) Bit4 –Ring In, Ring In event (0= off, 1= on) Bit5 – BtWake,Bluetooth wake up event(0=off,1=on) Bit6 – Dock ,Dock in event(0=off,1=on) |
| B2h | Percentage : Battery in critical low condition. |
| B3h | Percentage : Battery in low condition. |
| B4h | Fan1 pulse width low byte |
| B5h | Fan1 pulse width high byte |
| B6h | Fan2 pulse width low byte |
| B7h | Fan2 pulse width high byte |
| B8h | Bluetooth Status Bit0 – Detach(0=Detach,1=Attach) Bit1 – Power(0=power off,1=power on) Bit2 – Detach Status(0=Detach OK,1=Attach OK) Bit3 –Power Status(0=Power off OK,1=Power on OK) Bit4 – Switch(0=switch off,1=switch ok) Bit5 –Wake up |
| B9h | Lcd brightness value (0x00-0x0a) |
| BAh | Lcd contrast value (0x00-0x1F) |
| BBh | Device module status Bit0- 1=Wirless LAN active, 0=Wirless LAN no active Bit1- 1=BlueTooth active, 0=BlueTooth no active Bit2- 1=Wirless LAN exist, 0=Wirless LAN no exist Bit3- 1=BlueTooth exist, 0=BlueTooth no exist Bit4- 1=Kill switch on, 0=Kill switch off Bit 5-1=WireLess LAN initial OK |
| BCh | Project ID |
| BDh | CPU type |
| BEh | Reserved |
| BFh | Reserved |

### Control method for 1st battery pack RAM definition

| Offset | Description |
| --- | --- |
| C0h | Bit4-6 – Manufacturer Bit 6 5 4 0 0 1 : Sanyo 0 10 : Sony 1 0 0 : Panasonic Bit7- Battery type 0 : Ni-MH 1 : Li-ion |
| C1h | Battery Status Bit0 – Discharging Bit1 – Charging Bit2 – Discharging and Now is critical low Bit3-7 – Reserved |
| C2h-C3h | Remaining Capacity |
| C4h-C5h | Serial Number |
| C6h-C7h | Present Voltage |
| C8h-C9h | Design Voltage |
| CAh-CBh | Design Capacity |
| CCh-CDh | Full charge capacity |
| Ceh | Gasgauge |
| CFh-DFh | Reserved |

### Control method for 2nd battery pack RAM definition

| Offset | Description |
| --- | --- |
| E0h | Bit4-6 – Manufacturer Bit 6 5 4 0 0 1 : Sanyo 0 10 : Sony 1 0 0 : Panasonic Bit7- Battery type 0 : Ni-MH 1 : Li-ion |
| E1h | Battery Status Bit0 – Discharging Bit1 – Charging Bit2 – Discharging and Now is critical low Bit3-7 – Reserved |
| E2h-E3h | Remaining Capacity |
| E4h-E5h | Serial Number |
| E6h-E7h | Present Voltage |
| E8h-E9h | Design Voltage |
| EAh-EBh | Design Capacity |
| ECh-EDh | Full charge capacity |
| EEh | Gasgauge |
| EFh-F8h | Reserved |
| F9h | Real Fan Speed (Low Byte) |
| FAh | Real Fan Speed (High Byte) |
| EBh -FFh | Reserved |

Embedded Controller chipset

### NS PC87591

The Compact RISC CR16B is an advanced, general-purpose, 16-bit microprocessor core with a RISC architecture. The core is responsible for arithmetic and logic operations and program control.

### Force update PC87591

For 591-S version support internal rom,and it can force update EC.Press Fn+B+Q keys and then plug in AC,boot loader will force update 591 internal ROM from external ROM.