# Chapter 4: DC-DC CONVERTER

## DCL56 series Power System block diagram

### Description

This specification defines the performance and characteristic of 65W AC adapter power supply. It supplies a constant voltage 19V output source for DCL56 series notebook computer.

### Feature

 Accepts universal input from 90V AC to 264V AC

-  Offers constant Voltage 19V output source with 65W max output power capacity.

-  High efficiency 83% min at 115V AC

-  Compact Size

### Electrical Specification

| Input Voltage range | universal input, 90VAC TO 264AC |
| --- | --- |
| Inrush current | No damage shall occur and the input fuse shall not blow |
| Input frequency range | 47~63Hz |
| Input Current: | 1.7 Amax at 90VAC |
| Start-up time |  5sec Max. @115VAC |
| HOLD-UP time | 5ms min. @115V AC, full load condition |
| OVP | 29V max. automatic shut down |
| Short circuit protection | Output can be shorted without damage, and auto recovery |
| OUTPUT Voltage Regulation | 18.05-21V including the effects of line Voltage variation, load current, ripple and noise |
| OUTPUT Current | 0Amin, 3.42Amax continuous |
| OUTPUT Voltage ripple | 380mv PK-PK for resistor load |
| OUTPUT Voltage Dynamic regulation | Output voltage within 17.57V~21V, load current 10%100%, frequency is 100HE to 5kHz, 50% duty cycle. |
| DC OUTPUT PIN OUT |  PIN1 Center Pin Adapter +output  PIN2 Barrel (Ring) Adapter returns. |
| Temperature Range |  Operating temperature: 0 °C TO 40 °C  Storage temperature : -30 °C TO 85 °C |

CAUTION

Danger of explosion if battery is incorrectly replaced.

Replace only with the same or equivalent type recommended by the manufacturer.

Dispose of used batteries according to the manufacturer’s instructions.

# DC-DC CONVERTER

## Description

The DC-DC converter is designed to supply the power for ACL10 series notebook computer of Compal. It supply +5VALWP, +3VALWP, +12VALWP, +1.8VSP, +1.05VALWP,+1.5VALWP ,+1.2VSP,1.25VSP,2.5VP for logical system, +CPU_CORE, for CPU and supplies +3VALWP for the built-in NS87591 microprocessor which handles the keyboard and PMU control functions of the system. The power ON/OFF is controlled by NS87591. There is also a built-in charger power source. It can charge battery pack whether the computer is ON or OFF.

- Features

-  High efficiency, up to 85% (using battery)

-  Accept wide range DC input voltage from 12V to 20V

-  Built-in charger power source

-  The power ON/OFF is controlled by software

- Electrical specification

- Input Voltage/Current

 12V to 20V at the summing point of AC-DC and battery

 INPUT Current 5.1A max from battery

 INPUT Current 3.42A max from AC-DC Adapter.

Temperature Range

-  Operating temperature : 0° C to 40° C

-  storage temperature range : -20° C to 65° C

## DC/DC OUTPUT

### Fixed output voltage/Current

| Item | +5VALWP | +1.8VALWP | +CPU_COREP | +1.2VSP |
| --- | --- | --- | --- | --- |
| nominal voltage | +5V | +1.8V | +1.3V | +1.2V |
| min. current | 0A | 0A | 0A | 0A |
| max. current | 3.5A | 0.1A | 20A | 0.1A |
| peak current | 4.5A | 1A | 25A | 0.3A |
| total regulation | 5V±5% | 1.8V±5% | 1.3±0.1V | 1.2V+5% |
| ripple voltage | 100mVp-p max | 36mVp-p max | 50mVp-p max | 60mVp-p max |

| Item | +12VALWP | +3VALWP | +1.5VALWP | +2.5VP |
| --- | --- | --- | --- | --- |
| nominal voltage | +12V | +3.3V | +1.5V | +2.5V |
| min. current | 0A | 0A | 0A | 0A |
| max. current | 100mA | 4A | 1.0A | 5A |
| peak current | 100mA | 5A | 1.5A | 6A |
| total regulation | 12V±5% | 3.3V±5% | 1.5V±5% | +1.25V±5% |
| ripple voltage | 200mVp-p max | 100mVp-p max | 100mVp-p max | 50mVp-p max |

| Item | +1.25VSP | +1.8VSP | +1.05VALWP |
| --- | --- | --- | --- |
| nominal voltage | +1.25V | +1.8V | +1.05V |
| min. current | 0A | 0A | 0A |
| max. current | 0.5A | 1.5A | 0.1A |
| peak current | 1A | 2.5A | 1A |
| total regulation | 1.25V±2% | 1.8V±5% | 1.05V±5% |
| ripple voltage | 25mVp-p max | 50mVp-p max | 21mVp-p max |

- Charger

-  Controlled by ENEKB910 microprocessor from motherboard

-  Temperature sense capability for the battery

- (charge active between 0°C~ 50°C)

-  Fast charge 3.0Amps(typical.) for Li-Ion Battery when system off, approach 20W fast charge when system ON. (depend on system load)

-  When system turn off, the charged time from empty to full be 3.0 hrs typically for Li-ION battery.

-  Other battery services are presented by NS87591 microprocessor includes

- maximum charging timer, charging temperature range etc.

-  Charger power:

-  BATT+ Constant voltage mode: 16.8V±1% for Li-Ion battery.

-  Constant current mode: 3.0A±8%

-  Constant adapter current mode: 3.3A±6%

### OVER Current protection:

+5VALWP:           6.6~10.5A
+3VALWP:	     6.4~10.4A

+CPU_COREP: 45.6.A~60A

### OVER Voltage protection:

+5VALWP:	   5.51 ~6.00V

+3VALWP:	   3.53~ 3.84V

+CPU_COREP: 1.95 ~2.05V

### Under voltage protection

+5VALWP: 	   5V *(80%)

+3VALWP:	3.3V *(80%)

+CPU_COREP  +CPU_COREP Voltage * (65 ~75) %

### Short circuit protection:

Latch mode for +5VALWP, +3VALWP, +CPU_COREP

## I/O

### P1:AC adapter input Jack socket

 Pin 1: Center pin Adapter power + input

 Pin 2: Barrel (Ring) Adapter power return

### DC/DC interface

| Item | Description | I/O |
| --- | --- | --- |
| SUSP# | active low input, enable skip mode of TPS5120 | I |
| 51ON# | active low input, this pin controls VS output when system is using battery power, it is controlled by NS87591. | I |
| FSTCHG | active high input, when NS87591 judges the battery shall be charged, then this pin will be pulled high. | I |
| IREF | Battery change current control IREF= 1.31 * I change ,IREF= 0 ~ 3.3V | I |
| ACIN | active high output, when adapter present, this pin shall be pulled high, output for NS87591. | O |
| VGATE | power good output signal, active high, from +CPU_COREP turn on edge and goes low before falling –5% of +CPU_COREP. | O |
| VR_ON | Active high input, +VID(1.2V) on/off pin. | I |
| BATT_TEMPA | An analog signal, it offers the status of battery temperature for NS87591. | O |
| VID [0..5] | CPU VID Pin. It determinate the +CPU_COREP output voltage. | I |
| 6C/8C# | High:6 Cell Li-ION Battery, Low: 8 Cell Li-ION Battery | O |
| AB/I | Battery discharge active pin. | O |
| EC_SMD1 | Main battery SMBus data pin. | I/O |
| EC_SMC1 | Main battery SMBus clock pin. | I/O |
| MAINPWON | H/W Thermal protection | O |
| STP_CPU# | CPU MODE control pin , It comes from sorth bright | I |
| PM_DPRSLPVR | CPU MODE control pin , It comes from sorth bright | I |
| PSI# | CPU MODE control pin , It comes from sorth bright | I |

### BATTERY

#### Li-ion Smart battery for DCL56 series

#####  18650 2P4S 14.8V/4300mAH, Li-ion battery

#####  18650 1P4S 14.8V/2150mAH, Li-ion battery

#####  More than 300 charging/discharging cycles.

#####  Modularized battery pack, easy to be replaced.

#### On board RTC battery:Maxell ML1220T13RE /3.0V /14mAH Lithium.

#### Sanyo ML1220T28 /3.0V /15mAH Lithium

## DCL56 INVERTER SPECIFICATION

### Description

This inverter is designed to light up the CCFL of LCD for notebook.It should be supported following LCD panels.

-  DCL56 project

| No. | Supplier | Model | Type | Frequency (KHz) | Current (mA) | VS max. (at 0oC) (Vrms) | Work Voltage (Vrms) | Light nits (typ.) | DAC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | AU | B141XG05 | 14.1” XGA | 50/60/70 | 3/6/7 | 1500 | --/650/-- | 200 | 1 |
| 2. | Toppoly | TD141TGCD1 | 14.1” XGA | 50/60/80 | 3/6/6.5 | 1450 | --/660/-- | 160 | 1 |
| 3. | AU | B150XG01-V2 (HW2,FW1) | 15.0” XGA | 40/50/60 | 3/6/7 | 1150 | 625/690/750 | 180 | 1 |
| 4. | Hitachi | TX38D81VC1CAB | 15.0” XGA | 40/50/70 | 3/--/6 | 1600 | --/690/-- | 180 | 1.5 |
| 5. | LG | LP150X08-A5 | 15.0” XGA | 45/58/80 | 3/5/6 | 1370 | 655/685/805 | 150 | 1.5 |
| 6. | Hannstar | HSD150PX17-A | 15.0” XGA | 40/50/80 | 3/6/7 | 1600 | 580/640/700 | 200 | 1 |
| 7. | Sanyo | TM150XG-02L11 | 15.0” XGA | 40/--/65 | 3/--/6 | 1550 | --/670/-- | 190 | 1.5 |
| 8. | AU | B150XG02-V1 | 15.0” XGA | 40/50/60 | 3/6/7 | 1150 | --/700/-- | 200 | 1 |
| 9. | AU | B150PG01(PRC) | 15.0”SXGA | 50/60/70 | 3/5.5/7 | 1150 | --/700/-- | 150 | 1.5 |
| 10. | QDI | QD150XL06 | 15.0” XGA | 50/55/60 | 3/6/7 | 1500 | 594/660/726 | 160 | 1 |

There are two control signals that come from system to control lamp brightness. One signal is named DAC_BRIG, which limits current to meet LCD lamp current specification. Another one is named PWM, which adjusts lamp brightness. This inverter brightness is adjusted by PWM burst mode. The PWM burst mode is that turning on and off the lamp at a rate of 150Hz. The effective brightness is a function of the duty cycle.

### Features

-  Wide range 9V to 21V input voltage

-  Brightness adjustment by PWM burst mode.

-  Close loop controls lamp current.

### Absolute maximum rating

- Environment: Temperature: Operating temperature: 0℃ ~ 55℃

 Storage temperature : -20℃ ~ 70℃

 Humidity:  0 ~ 90% without condensation

 MTBF:  MIN 50000 hours.

### Electrical characteristic

| No | Item | Symbol | Min. | Typ. | Max. | Unit | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Input voltage | INV_PWR | 9 | 14.8 | 21 | V | 7.5V (contimuous) can work |
| 2 | Input current | Iin | -- | 0.33 | -- | A |  |
| 3 | Lamp current | IL | 3.0 | -- | 6.8 | mA | DAC=0V *Note 2 |
| 4 | Lamp current | IL | 2.7 | -- | 6.3 | mA | DAC=1V |
| 4 | Frequency | F | 45 | 55 | 60 | KHz | *Note3 |
| 5 | Output power | Pout | -- | -- | 4.5 | W |  |
| 6 | Efficiency |  | 80% | -- | -- | -- |  |
| 7 | OP-Lamp voltage | Vs | 1600 | -- | -- | V | At 0oC |
| 8 | Starting time | Tvs | 1 | -- | 1.5 | Sec |  |
| 9 | Dispoff# |  | 2.8 | 3.3 | 3.6 | V | Backlight on/off signal |
|  |  |  | 0 | 0.5 | 0.8 | V | Low level |
| 10 | Limited lamp maximum current | DAC-BRIG | 0 |  | 3.3 | V | *Note 2 |
| 11 | PWM signal *note 4 | INV_PWM | 142 | 150 | 158 | Hz | PWM signal frequency |
|  |  |  | 3.0 | 3.3 | 3.6 | V | PWM signal amplitude |
|  |  |  | 30 | -- | 100 | % | Duty=TonPeriod |
| 12 | lamp current over-shoot | IZero−PK | -- | -- | 10 | % | Line transient ( 10.8V to 21V/100us) and turn on transient |
| 13 | Current Waveform factor | IpIrms | 1.27 | 2 | 1.56 | Multiple | OR I−pIrms *10 |
| 14 | Unbalance Rate | Ip−I−pIrms | -10% | 0 | +10% | Multiple |  |
| 15 | Turn off current (Hight side) | IHL | -- | -- | 0 | A | PWM=30% |
| 16 | Turn off voltage (Low side) | Voff | -- | -- | 150Vp-p | V | PWM=30% |
| 17 | Voltage Rise time (Low side) | Trise | -- | -- | 300us | us | PWM=30% |
| 18 | Voltage fall time(Low side) | Tfall | -- | -- | 300us | us | PWM=30% |

Notes:

 The inverter can work in 7.5V input voltage (continuous), but 7.5V electron charactenstic will not be care.

 Limited lamp maximum current by DAC_BRIG signal:

When DAC_BRIG voltage is 0V and INV_PWM enables (100%), lamp has max. limited current.

When DAC_BRIG voltage is 3.3V and INV_PWM enables (100%), lamp has min. limited current.

When add 1V DAC, the 100% Lamp current will decrease 0.5mA.

DAC_BRIG signal comes from system chipset with internal resistance of 3KΩ.

 Inverter operating frequency should be within specification (45~60kHz) at max. and min. brightness load.

 INV_PWM enable implies INV_PWM signal is High level (On duty cycle is 100%). It is a square wave of 150Hz to adjust backlight brightness that is a function of PWM duty cycle. Backlight brightness is maximum value under INV_PWM at 100% and brightness is minimum under INV_PWM at 30%.

 The system interface signals belong to 3.3V.

 Please make sure open lamp output voltage should be within starting voltage specification.

 Inverter should pass human body safety test.

 Inverter should no smoking by any component open/ short test

 Transformer voltage stress should not be over 85% under any condition

( turn on overshoot transient and line transient).

 Audio noise should be less than 36dB at 10 cm distance.

# Electrical specification

### Electrical specification

| No | Symbol | Min. | Typ. | Max. | Unit | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Vopen. | -- | 650 | -- | Vrms | Lamp operating voltage(650+/-50) |
|  | IL | 6.2 | 6.5 | 6.8 | mArms | DAC_BRIG: 0 V, PWM: 100% |
|  | IL | 3.0 | 3.3 | 3.6 | mArms | DAC_BRIG: 0 V, PWM:30% |
|  | IL | 5.7 | 6 | 6.3 | mArms | DAC_BRIG: 1 V, PWM: 100% |
|  | IL | 2.7 | 3 | 3.3 | mArms | DAC_BRIG: 1 V, PWM:30% |
|  | f | 45 | 55 | 60 | KHz |  |
|  |  | 80% | -- | -- | -- |  |

### Thermal

All components on inverter board should follow below rules:

 Component using conditions (component stress) must be within component specification including voltage rating, current rating, temperature etc.

 Component temperature should follow below:

 Δ T < 30℃ , at 25 , 35℃.

 Component temperature should be less than 70℃ inside system at 35℃.

### Connector description

- Input Connector:

 CN1:ACES 87213-0700: JST SM07B-SRSS-TB

| Pin No. | Symbol | Description |
| --- | --- | --- |
| 1 | INV_PWR | Input voltage(9V-21V)) |
| 2 | INV_PWR | Input voltage(9V-21V)) |
| 3 | INV_PWM | Adjust brightness by burst mode |
| 4 | DISOFF # | Backlight on/off control, active HIGH(3.3V) |
| 5 | DAC_BRIG | Max. current limit |
| 6 | GND | Power system return |
| 7 | GND | Power system return |

#### Output Connector:

  CN2: JST_SM02B_BHSS-1

| Pin No. | Symbol | Description |
| --- | --- | --- |
| 1 | HV | Connected to high voltage of LCD lamp |
| 2 | LV | Connected to low voltage of LCD lamp |

Note :

Please mark “ CAUTION HIGH VOLTAGE” around CN2

### Safety Protection

#### Open lamp protection:

When inverter is on open lamp status, any component on inverter should be O.K and inverter is no damaged, no fire and no arcing. If inverter can’t shunt down during open lamp happen, inverter must pass below conditions:

 Human body test.

 Open lamp burning: Inverter burns for 24 hours at open lamp status. No parts damage.

#### Human body safety test:

Short inverter output, transformer secondary output to GND by a 2KΩ resistor which connects one end to GND and another one to those outputs. They should meet output current limitation requirement as follow. Output current I is the current that flows through 2KΩ resistor.

 Output current  I ≦ 0.7mA , if frequency f ≦ 1KHz

 Output current  I ≦ 0.7mA * f (kHz) , if f ≧ 1KHz.

However, output current should be less than 70mA even frequency is more than 100KHz.

#### Abnormal test:

Any one component is short or open; inverter should be no fire, no arcing. And result must meet output current limitation requirement.