SCOPE

Chapter 1: System Description

This document describes the functional specifications for the Compal NoteBook personal computer ACY23 series. The system is hardware and software compatible with the IBM PC/AT personal computer.

## CPU

- Intel Celeron 1.2GHZ/PIII 800MHZ or above

- FC-PGA2 package CPU

## Chip Set

- VIA VT8606 system controller.

- VIA VT686B for PCI to ISA Bridge with integrated super-I/O,sound Blaster /Direct 
       sound AC97 Audio, PCI IDE controller, Power Management Logic.

- NS87570 for Keyboard Controller, Battery management Unit, and RTC.

- OZ6933 for Card Bus PCMCIA controller.

- Integrated Savage 4 AGP 4X Graphics core on VT8603.

- CS4299 for AC97 CODEC.

- OZ163 for Direct CD play controller.

- TI TSB43AB22 for 1394 OHCI PHY/LINK Layer Controller

## Memory

- 128MB on board.

- On board with one 144-pin +3.3V SO-DIMM connector, supporting, SDRAM 
       memories card. Maximum upgradable to 320MB by one 256 SO-DIMM module.

- 128 KB/256KB L2 Cache on CPU.

## BIOS

- 512KB Flash ROM for system BIOS.

- Suspend to RAM/Disk.

- Password protection (System).

- Windows 98 ready with PnP

- Various hot key for system control.

- Refreshable

- Complete ACPI 1.0B Function

## Power

-  cells Li-Ion 18650 size smart battery Pack with 57Wh capacity

- hours operation as running battery mark 4.0 program

- max4.5Hours  charging time to 100% capacity on Li-Ion Battery (System on)

## One 2.5",9.5mm Hard Disk, support with 10GB/5GB/20GB/30GB

- Bus Master IDE

- 9.5m/m, 2.5”HDD Support

- Support Ultra 100 synchronous DMA(ATA-100)

## FDD/CD-ROM devices

- 12.7mm, 1.44MB FDD device

- 5.25” 12.7mm height CD-ROM device

- 5.25” 12.7mm height DVD-ROM device

- 5.25” 12.7mm height CD-RW device

- 5.25” 12.7mm height CD-RW,DVD –ROM device

## Options

- 64MB/128MB/256MB SO-DIMM modules

- MINI PCI Modem module

- MINI PCI Modem/LAN Combo module

## Touch PAD with two buttons

- Left Button

- Right Button

## I/O Ports

- One 25 pins Parallel port, EPP/ECP Capability

- One 15 pins CRT port, Support DDC 2B

- One 6 pins external PS2/AT full keyboard connector & One TV-out connector

- One MIC In port

- One headphone-out with SPDIF port

- One 2pins AC Adapter Jack

- One type III or two type II PCMCIA Card Bus slots

- Two 4 pins USB ports

- One 4 pins 1394 port

- One RJ11/RJ45 Port

## PCMCIA Card Organization

- One type III card socket or two type II card sockets

- SRAM, OTPROM, FLASH ROM, mask ROM memory card up to 64MB

- MODEM/LAN card

- 1.8" 10.5mm removable ATA Device

- Card bus card

- ACPI 1.0 Compliant

## Excellent Power Management Function

- Standby mode, Suspend to RAM or Suspend to Disk mode, by time out or by hot
        key

- HDD Local Stand-By mode by time out

- LCD Local Stand-By mode by time out

- Low battery alarm by beep

- Auto-backlight off when LCD cover closed

- Full ACPI 1.0B supported

- LCD Auto-DIM mode by time out

## Switch

- Power switch

- Lid switch

- Application/ Button Lock/ CD-Play Switch

- Eject/ Stop Button

- Play/ Pause Button

- Reverse Button

- Forward Button

- Volume Down Button

- Volume Up Button

## AC Adapter

- Universal AC adapter module. 90-264V AC, 47-63HZ, 60W.

## Security

- Boot-up password protection

- Single level password architecture. (Supervisor)

## Memory Card & SO-DIMM Socket

- 64/128/256MB +3.3V JEDEC Standard 144-pins SDRAM SO-DIMM Memory module upgradable.

# Electrical Specifications

## Mother Board

## Microprocessor

- Intel  Celeron 1.2GHZ or above FC-PGA2 packing

## System Logic

- VIA VT8606Host Bridge/controller processor host Bus support

- Integrated DRAM controller

- PCI bus Interface

- Integrated Savage 4 graphics core

- Power Management Functions

- Supporting I/O Bridge

## Memory

- System DRAM

- 128MB on board SDRAM memory

- One JEDEC standard 144-pins SO-DIMM memory support +3.3V 128/256MB.

- System & KB Combine ROM BIOS

- 512KB Flash ROM

## Fixed Disk Interface

- VIA VT82C686B

## Video Subsystem

### Integrated Savage4 2D/3D/Video Accelerator

- VIA VT8606 integrated Savage4  graphic core.

- Optimized Shared Memory Architecture(SMA).

- 8/16/32MB frame buffer using system memory.

- Floating point triangle setup engine.

- Single cycle 128-bit 3D architecture.

- 8M triangles/second setup engine.

- 140M pixels/second trilinear fill rate.

- Full internal AGP 4x performance.

- S3 DX7 texture compression(S3TC)TM.

- Next generation, 128-bit 2D graphics engine.

- Hight quality DVD video playback.

- Flat panel monitor support.

- 2D/3D resolutions up to 1920x1440.

### 3D Rendering Features

- Single-pass multiple textures.

- Anisotropic filtering.

- 8-bit stencil buffer.

- 32-bit true color rendering.

- Specular lighting and diffuse shading.

- Alpha blending modes .

- Massive 2K x2K textures.

- MPEG-2 video textures.

- Vertex and table fog.

- 16 or 24-bit Z-buffering.

- sprite anti-aliasing ,reflection mapping, texture morphing

- shadows ,procedural textures and atmospheric effects.

### 2D Hardware Acceleration Features

- ROP3 Ternary Raster Operation BitBLTs.

- 8,16, and 32 bpp mode acceleration

### Motion Video Architecture

- Hight quality up/down scaler.

- Planar to packed format conversion.

- Motion compensation for full speed DVD playback.

- Hardware subpicture blending and highlights.

- Multiple video windows for video conferencing.

- Contrast, hue, saturation, brightness and gamma controls.

- Digital port for NTSC/PAL TV encoders.

### Extensive LCD Support

- 36-bit DSTN/TFT flat panel interface with 256 gray shade support.

- Integrated 2-channel 110MHZ LVDS interface.

- Support for all resolutions up to 1600x1200.

- ZV-Port interface.

- Panel power sequencing .

- Hardware Suspend/Standby control.

### Flat Panel Monitor Support

- 12-bit TFT flat panel interface to TMDS encoders.

- Digital Visual Interface(DVI) 1.0 compliant.

### Concurrent PCI Bus Controller

- PCI 2.2 compliant,32-bit 3.3V PCI interface with 5V tolerant inputs.

- Supports up to 5 PCI masters.

- PCI to system memory data streaming support.

- Delay transaction from PCI master accessing DRAM.

- Symmetric arbitration between Host/PCI bus for optimized system

- performance.

### Advanced System Power Management Support

- Dynamic power down of SDRAM(CKE).

- Independent clock stop controls for CPU/SDRAM,AGP,and PCI bus.

- PCI and AGP bus clock run and clock generation coutrol.

- VTT suspend power plane preserves memory data.

- Suspend-to-DRAM and self-refresh power down.

- Low-leakage I/O pads.

- ACPI 1.0b and PCI Bus power Management 1.1 compliant.

### Full Software Support

- Drivers for major operating system and APIs:[Windows® 9x,Windows

- NT4.0,Windows 2000,Direct3DTM,Direct Draw and Direet shoTN

- openGL TN , ICD for Windows 9x ,NT and 2000

- North Bridge/Chipset and Video BIOS support.

### Additional Features

- 250MHz RAMDA with Gamma Correction.

- 12-bit interface to external TV encoder.

- I 2C Serial Bus and DDC Monitor Communications.

- 2.5V Core and Mixed 3.3V/5V Tolerant and GTL+I/O.

- 35x35mm PBGA package with 552 balls.

## VT82C686B integrated Super IO controller

### Integrated Super IO Controller

- Supports 2 serial ports ,IR port, parallel port, and floppy disk controller

- functions

- Two UARTs for Complete Serial Ports

- Programmable character lengths(5,6,7,8)

- Even, odd, stick or no parity bit generation and detection Programmable

- baud rate generator

- Independent transmit/receiver FIFOs Modem Control

- Plug and play with 96 base IO  address and 12 IRQ options.

- Infrared-IrDA (HPSIR) and ASK (Amplitude Shift Keyed) IR port

- multiplexed on COM2.

- Multi-mode parallel port

- Standard mode, ECP and EPP support

- Plug and with 192 base IO address,12 IRQ and 4 DMA options

- Floppy Disk Controller

- 16bytes of FIFO Data rates up to 1 Mbps

- Perpendicular recording drive support

- Two FDDs with drive swap support

- Plug and play with 48 base IO ads address,12 IRQ and 4 DMA option

### Keyboard Controller

- PC87570 is use as keyboard controller and battery management unit

### Audio subsystem

#### VIA VT82c686B integrated Sound Blaster/Direct Sound AC97 Audio

- Dual full-duplex Direct Sound channels between system memory and AC97 link.

- PCI master interface with scatter/gather and bursting capability.

- 32 byte FIFO of each direct sound channel.

- Host based sample rate converter and mixer.

- Standard v1.0 or v2.0 AC97 Codec interface for single or cascaded AC97 Codec 
        from multiple vendors

- Hardware SoundBlaster Pro for Windows DOS box and real-mode DOS legacy 
       compatibility.

- Plug and play with 4 IRQ,4 DMA, and 4 I/O space options for SoundBlaster Pro and 
       MIDI hardware

- Hardware assisted FM synthesis for legacy compatibility.

- Direct two game ports and one MIDI port interface.

- Cirrus logic C4299 for AC97 codec

- Ac ,97 2.1 Compatible.

- Industry Leading Mixed Signal Technology.

- 20-bit Stereo Digital-to-Analog Converters.

- 18-bit Stereo Analog-to- Digital Converters.

- Sample Rate Converters.

- Four Analog Line-level Stereo Inputs for LIN_IN,CD,VIDEO, and AUX.

- Two  Analog Line-level Mono Inputs for Modem and Internal PC Beep.

- Dual Stereo Line-level Outputs for LINE_OUT and ALT_LINE _OUT.

- Dual Microphone Inputs.

- High Quality Pseudo-Differential CD Input.

- Extensive Power Management Support.

- Meets or Exceeds the Microsoft® PC 99 Audio Performance Requirements.

- S/PDIF Digital Audio Output.

- CrystalClear ® 3D Stereo Enhancement.

### IEEE 1394A Host Controller

- TI TSB43AB22 1394a-2000 OHCI PHY/Link layer Controller.

- Fully Supports provisions of IEEE 1394-1995 standard for high-performance serial 
       bus and the IEEE 1394a-2000 supplement.

- Fully interoperable with FireWireTM and i.LINKTM implementations of IEEE Std 1394 .

- Meets IntelTM Mobile Power Guideline 2000.

- Full IEEE 1394a-2000 support includes: connection debounce, arbitrated short 
       reset, multispeed concatenation, arbitration acceleration, fly-by concatenation, and 
       port disable/suspend/resume.

- Power-down features to conserve energy in battery-powered applications include: 
       automatic device power down during suspend, PCI power management for link-
       layer, and inactive ports powered down.

- Ultralow-power sleep mode.

- Provides two IEEE 1394a-2000 full compliant cable port at 100/200/400 megabits 
       per second (Mbits/s).

- Cable ports monitor line conditions for active connection to remote node.

- Cable power presence monitoring.

- Separate cable bias (TPBIAS) for each port.

- 3.3-V core logic with universal PCI interfaces compatible with 3.3-V and

- 5-V PCI signaling environments.

- Physical write posting of up to three outstanding transactions.

- Implements PCI burst transfers and deep FIFOs to tolerate large host latency.

- PCI_CLKRUN protocol.

- External cycle timer control for customized synchronization.

- Extended resume signaling for compatibility with legacy DV components.

- PHY-Link logic performs system initialization and arbitration functions.

- PHY-Link encode and decode function included for data-strobe bit level encoding.

- PHY-Link incoming data resynchronized to local clock.

- Low-cost 24.576-MHz crystal provides transmit and receive data at 100,200,and 400 
       Mbits/s.

- Node power class information signaling for system power managemanent.

- Serial ROM interface supports 2-wire serial EEPROM devices.

- Provides two general-purpose I/Os .

- Register software control of contender bit, power class bits, link active control bit, 
       and IEEE 1394a-2000 features.

- Fabricated in advanced low-power CMOS process.

- PCI and CardBus implementations.

- Isochronous receive dual-buffer mode.

- Out-of order pipelining for asynchronous transmit requests.

- Register access fail interrupt when PHY SYSCLK is not active.

- PCI power-management D1 power state.

### Audio DJ Controller

- O2 Micro OZ163

- “CDPlayer Mode” allows music CD play while notebook is OFF

- “Direct mode” with “Push mode” technology allows function buttons    
        (PLAY_PAUSE, NEXT_TRK/SCAN_FW, PREV_TRK/SCAN_RW, STOP_EJECT) 
        to work while system is ON

- Low power consumption – conserve notebook’s battery capacity

- Work hand-in-hand with traditional ISA/PCI based audio chipset

- “Pass through” mode allows this chip to be transparent to the OS and   system 
        hardware

- ATAPI compliant during CDPlayer Mode

- Auto power management-SLEEP and SUSPEND states

- Supports Volume up/down control through and digital volume controller

- Support activity icon display

- Supports track number display

- Minimal software required in Direct Mode

- Maintains High Quality sound

- Patent pending

### OZ6933 Card Bus Host Adapter

- Single-chip Card Bus host adapter

- Supports 2 PCMCIA 2.1 and JEIDA 4.2 R2 cards or 2 CardBus Cards.

- Support multiple FIFOs for PCI/CB data transfer

- Supports up 33MHz clock to CardBus Cards.

- Compliant with PCI spec. V2.2S, 2000 PC Card Standard 7.1and JEIDA 4.1

- ACPI-PCI Bus power management interface Specification Rev1.1 Comliant

- 3V card protection during host system suspend mode with Auto Card VS#re-sensing

- Supports both 5V and 3.3V PC cards

- Supports PCMCIA_ATA Specification

- No buffers required for PC Card socket interface

- PC99 Compliant

### Floppy Disk Drive.

- 3.5” 1.44MB, FDD device

### Hard Disk Drive

- 2.5" ,9.5mm height, up to 30GB

### Pointing Device

- Integrated Touch PAD with two buttons.

### CD-ROM devices module

- 24X CD-ROM upgradable

## Audio Port

### MIC IN

- AC-coupled input,100mVP-P maximum

### Headphone

- 1VP-P

### Built-in Speakers

- 8 (23x5.3m/m), 1 W

## Display Device

- COLOR TFT/XGA LCD (Unipac UB133X01-06)
Dimensions : 284.5 (W) X 214.5(H) X 5.8(D) mm (max)
Active area : 270.3(W) X 202.7(H) mm, 13.3"
1024 X 768 XGA Resolution
Response time: 50ms (max)
Contrast ratio 200:1 (Typ)
Brightness 130 Nit (Typ)

- COLOR TFT/XGA LCD (Sanyo TM133XG-02L06C)
Dimensions : 284.0 (W) X 214.5 (H) X 5.9 (D) mm (max)
Active area : 270.3(W) X 202.7(H) mm, 13.3"
1024 X 768 XGA Resolution
Response time: 42 (max)
Contrast ratio 250:1 (Typ)
Brightness 150 Nit (Typ)

- COLOR TFT/XGA LCD (IBM ITXG77X)
Dimensions : 298.5 (W) X 226.5 (H) X 6.1 (D) mm (max)
Active area : 285.7(W) X 214.3(H) mm, 14.1"
1024 X 768 XGA Resolution
Response time: 60 (max)
Contrast ratio 250:1 (Typ)
Brightness 150 Nit (Typ)

- COLOR TFT/XGA LCD (Hannstar HSD141PX11-A Rev.1)
Dimensions : 298.5 (W) X 226.5 (H) X 6.0 (D)mm (max)
Active area : 285.696(W)X214.272(H)mm,14.1”
1024 X 768 XGA Resolution
Response time: 50(max)
Contrast ratio 250:1 (Typ)
Brightness 140 Nit (Typ)

- COLOR TFT/XGA LCD (LG LP150X1-G2CP)
Dimensions : 315.5(W) X 242.3 (H) X 7.3(D)mm (max)
Active area : 304.1(W)X228.1(H)mm,15.0”
1024 X 768 XGA Resolution
Response time: 80(max)
Contrast ratio 300:1 (Typ)
Brightness 155 Nit (Typ)

- COLOR TFT/XGA LCD (IBM ITSX95C)
Dimensions : 317.3 (W) X 242. (H) X 6.3 (D) mm (max)
Active area : 304.5(W) X 228.3(H) mm, 15.0"
1400 X 1050SXGA Resolution
Response time: 60 (max)
Contrast ratio 200:1 (Typ)
Brightness 140 Nit (Typ)

## Keyboard

- 87 keys with 101/102 key emulation

- 1 Windows Keys, 1 Application Key

- Standard pitch, 3.0 mm travel length

- Palm-rest