Major Sub-assembly Specification

Chapter 3: Hardware

## System interconnection (For ACY23)

## MOTHER BOARD

### JP18 KBD/PS2_6.PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | KBD_DATA | 2. | EXT_DATA |
| 3. | GND | 4. | PS/2_VCC |
| 5. | KBD_CLK | 6. | EXT_CLK |

### JP13,JP15 USB _CON..PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | USB_UCCA | 2. | USB0_D- |
| 3. | USB0_D+ | 4. | USB_AGND |

### JP1 DSUB-9.PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | +5V | 2. | GND |
| 3. | RXD0F# | 4. | TXD0F# |
| 5. | DSR0F | 6. | RTS0F |
| 7. | CTS0F | 8. | DTR0F |
| 9. | RI0F | 10. | D0F |

### JP9 CRT CONN..PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | CRTR | 2. | CRTG |
| 3. | CRTB | 4. | NC |
| 5. | GND | 6. | GND |
| 7. | GND | 8. | GND |
| 9. | CRTVCC | 10. | GND |
| 11. | NC | 12. | DDCD |
| 13. | HSYNC | 14. | VSYNC |
| 15. | DDCC |  |  |

### JP10 LPTCN-25.PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | +5V_PRN | 2. | FD0 |
| 3. | FD1 | 4. | FD2 |
| 5. | FD3 | 6. | FD4 |
| 7. | FD5 | 8. | FD6 |
| 9. | FD7 | 10. | ACK# |
| 11. | BUSY | 12. | PE |
| 13. | SLCT | 14. | AFD/3M# |
| 15. | ERR# | 16. | INIT# |
| 17. | SLCTIN# | 18. | GND |
| 19. | GND | 20. | GND |
| 21. | GND | 22. | GND |
| 23. | GND | 24. | GND |
| 25. | GND |  |  |

### JP8 S CONN..PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | GND | 2. | N.C. |
| 3. | GND | 4. | LUMA |
| 5. | NC | 6. | CRMA |
| 7. | COMPS |  |  |

### JP22 HDD

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | PIDERST# | 2. | GND |
| 3. | PBD7 | 4. | PBD8 |
| 5. | PBD6 | 6. | PBD9 |
| 7. | PBD5 | 8. | PBD10 |
| 9. | PBD4 | 10. | PBD11 |
| 11. | PBD3 | 12. | PBD12 |
| 13. | PBD2 | 14. | PBD13 |
| 15. | PBD1 | 16. | PBD14 |
| 17. | PBD0 | 18. | PBD15 |
| 19. | GND | 20. | NC |
| 21. | PBDREQ | 22. | NC |
| 23. | PBIOW# | 24. | GND |
| 25. | PBDIOR# | 26. | GND |
| 27. | PBIORDY | 28. | PCSEL |
| 29. | PBDACK# | 30. | GND |
| 31. | IRQ14 | 32. | NC |
| 33. | PBA1 | 34. | NC |
| 35. | PBA0 | 36. | PBA2 |
| 37. | PBCS1# | 38. | PBC3# |
| 39. | HDD_LED# | 40. | GND |
| 41. | +5VS | 42. | +5VS |
| 43. | GND | 44. | NC |

### JP12 1394.PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | 10TPBO | 2. | TPBO+ |
| 3. | TPAO- | 4. | TPAO+ |
| ### JP3 HEADERX.PRT |  |  |  |
| PIN NO | SIGNAL | PIN NO | SIGNAL |
| 1. | +5VALW | 2. | ACIN |
| 3. | HDD_LED# | 4. | CD_FDD_LED# |
| 5. | CAPS_LED# | 6. | ARROW_LED# |
| 7. | NUM_LED# | 8. | GND |
| 9. | +5VS | 10. | +3VS |
| ### JP20 144 SODIMMI.PRT |  |  |  |
| PIN NO | SIGNAL | PIN NO | SIGNAL |
| 1. | GND | 2. | GND |
| 3. | MD0 | 4. | MD8 |
| 5. | MD1 | 6. | MD9 |
| 7. | MD2 | 8. | MD10 |
| 9. | MD3 | 10. | MD11 |
| 11. | +3V | 12. | +3V |
| 13. | MD4 | 14. | MD12 |
| 15. | MD5 | 16. | MD13 |
| 17. | MD6 | 18. | MD14 |
| 19. | MD7 | 20. | MD15 |
| 21. | GND | 22. | GND |
| 23. | RCAS#0 | 24. | RCAS#1 |
| 25. | RCAS#4 | 26. | RCAS#5 |
| 27. | +3V | 28. | +3V |
| 29. | MMA0 | 30. | MMA3 |
| 31. | MMA1 | 32. | MMA4 |
| 33. | MMA2 | 34. | MMA5 |
| 35. | GND | 36. | GND |
| 37. | MD32 | 38. | MD40 |
| 39. | MD33 | 40. | MD41 |
| 41. | MD34 | 42. | MD42 |
| 43. | MD35 | 44. | MD43 |
| 45. | +3V | 46. | +3V |
| 47. | MD36 | 48. | MD44 |
| 49. | MD37 | 50. | MD45 |
| 51. | MD38 | 52. | MD46 |
| 53. | MD39 | 54. | MD47 |
| 55. | GND | 56. | GND |
| 57. | NC | 58. | NC |
| 59. | NC | 60. | NC |
| 61. | CLK_SDRAM2 | 62. | CKE2 |
| 63. | +3V | 64. | +3V |
| 65. | SRASA# | 66. | SCASA# |
| 67. | RMWEA# | 68. | CKE3 |
| 69. | RRAS#2 | 70. | MMA14 |
| 71. | RRAS#3 | 72. | NC |
| 73. | GND | 74. | CLK_SDRAM3 |
| 75. | GND | 76. | GND |
| 77. | NC | 78. | NC |
| 79. | NC | 80. | NC |
| 81. | +3V | 82. | +3V |
| 83. | MD16 | 84. | MD24 |
| 85. | MD17 | 86. | MD25 |
| 87. | MD18 | 88. | MD26 |
| 89. | MD19 | 90. | MD27 |
| 91. | GND | 92. | GND |
| 93. | MD20 | 94. | MD28 |
| 95. | MD21 | 96. | MD29 |
| 97. | MD22 | 98. | MD30 |
| 99. | MD23 | 100. | MD31 |
| 101. | +3V | 102. | +3V |
| 103. | MMA6 | 104. | MMA7 |
| 105. | MMA8 | 106. | MMA11 |
| 107. | GND | 108. | GND |
| 109. | MMA9 | 110. | MMA12 |
| 111. | MMA10 | 112. | MMA13 |
| 113. | +3V | 114. | +3V |
| 115. | RCAS#2 | 116. | RCAS#3 |
| 117. | RCAS#6 | 118. | RCAS#7 |
| 119. | GND | 120. | GND |
| 121. | MD48 | 122. | MD56 |
| 123. | MD49 | 124. | MD57 |
| 125. | MD50 | 126. | MD58 |
| 127. | MD51 | 128. | MD59 |
| 129. | +3V | 130. | +3V |
| 131. | MD52 | 132. | MD60 |
| 133. | MD53 | 134. | MD61 |
| 135. | MD54 | 136. | MD62 |
| 137. | MD55 | 138. | MD63 |
| 139. | GND | 140. | GND |
| 141. | SDADIMMI | 142. | SCKDIMMI |
| 143. | +3V | 144. | +3V |

### JP17 124-MINIPCI.PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | NC | 2. | NC |
| 3. | NC | 4. | NC |
| 5. | NC | 6. | NC |
| 7. | NC | 8. | NC |
| 9. | NC | 10. | NC |
| 11. | NC | 12. | NC |
| 13. | NC | 14. | NC |
| 15. | NC | 16. | NC |
| 17. | PIRQB# | 18. | +5VS |
| 19. | +3VS_MINIPCI | 20. | PIRQD# |
| 21. | NC | 22. | NC |
| 23. | GND | 24. | 3.3VAUX |
| 25. | PCLK_MDM | 26. | PCIRST# |
| 27. | GND | 28. | +3VS_MINIPCI |
| 29. | REQ#1 | 30. | GNT#1 |
| 31. | +3VS_MINIPCI | 32. | GND |
| 33. | AD31 | 34. | MDMPME# |
| 35. | AD29 | 36. | NC |
| 37. | GND | 38. | AD30 |
| 39. | AD27 | 40. | +3VS_MINIPCI |
| 41. | AD25 | 42. | AD28 |
| 43. | NC | 44. | AD26 |
| 45. | C/BE# | 46. | AD24 |
| 47. | AD23 | 48. | IDSEL |
| 49. | GND | 50. | GND |
| 51. | AD21 | 52. | AD22 |
| 53. | AD19 | 54. | AD20 |
| 55. | GND | 56. | PAR |
| 57. | AD17 | 58. | AD18 |
| 59. | C/BE#2 | 60. | AD16 |
| 61. | IRDY# | 62. | GND |
| 63. | +3VS_MINIPCI | 64. | FRAME# |
| 65. | CLKRUN# | 66. | TRDY# |
| 67. | SERR# | 68. | STOP# |
| 69. | GND | 70. | +3VS_MINIPCI |
| 71. | PERR# | 72. | DEVSEL# |
| 73. | C/BE#1 | 74. | GND |
| 75. | AD14 | 76. | AD15 |
| 77. | GND | 78. | AD13 |
| 79. | AD12 | 80. | AD11 |
| 81. | AD10 | 82. | GND |
| 83. | GND | 84. | AD9 |
| 85. | AD8 | 86. | C/BE#0 |
| 87. | AD7 | 88. | +3VS_MINIPCI |
| 89. | +3VS_MINIPCI | 90. | AD6 |
| 91. | AD5 | 92. | AD4 |
| 93. | NC | 94. | AD2 |
| 95. | AD3 | 96. | AD0 |
| 97. | +5VS | 98. | NC |
| 99. | AD1 | 100. | NC |
| 101. | GND | 102. | GND |
| 103. | VGA_ACSYNC | 104. | NC |
| 105. | VGA_ACSDATAI | 106. | VGA_ACSDATAO |
| 107. | VGA_ACBITCLK | 108. | NC |
| 109. | NC | 110. | VGA_ACRST# |
| 111. | BEEP | 112. | NC |
| 113. | AGND | 114. | GND |
| 115. | MD_MIC | 116. | MD_SPK |
| 117. | AGND | 118. | AGND |
| 119. | AGND | 120. | AGND |
| 121 | MINI_RI# | 122. | NC |
| 123 | +5VS | 124. | 3.3VAUX |

### SW2 POWER.-BTN

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | ON/OFF BTN# | 2. | ON/OFF BTN# |
| 3. | GND | 4. | GND |

### PJP1 BATT-B.PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | BATT+ | 2. | VBS |
| 3. | BN1/1LI# | 4. | TS |
| 5. | EEPROMVCC | 6. | BSCL |
| 7. | BQ_BATT/SMD | 8. | GND |

### U38 PGA370

| PIN NO | SIGNAL | PIN NO | SIGNAL |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A3 | D29# | AD34 | GND |  |  |  |
| A5 | D28# | AD36 | Vcc1.53 |  |  |  |
| A7 | D43# | AE1 | A17# |  |  |  |
| A9 | D37# | AE3 | A22# |  |  |  |
| A11 | D44# | AE5 | VccCORE |  |  |  |
| A13 | D51# | AE33 | A20M# |  |  |  |
| A15 | D47# | AE55 | IERR# |  |  |  |
| A17 | D48# | AE37 | FLUSH# |  |  |  |
| A19 | D57# | AF2 | VccCORE |  |  |  |
| A21 | D46# | AF4 | A35# |  |  |  |
| A23 | D53# | AF6 | A25 |  |  |  |
| A25 | D60# | AF32 | GND |  |  |  |
| A27 | D61# | AF34 | VccCORE |  |  |  |
| A29 | DEP7# | AF36 | GND |  |  |  |
| A31 | DEP3# | AG1 | EDGCTRL5 |  |  |  |
| A33 | DEP2# | AG3 | A19# |  |  |  |
| A35 | PRDY# | AG5 | GND |  |  |  |
| A37 | GND | AG33 | INIT# |  |  |  |
| AA1 | A27# | AG35 | STPCLK# |  |  |  |
| AA3 | A30# | AG37 | IGNNE# |  |  |  |
| AA5 | VccCORE | AH2 | GND |  |  |  |
| AA33 | VTT4 | AH4 | RESET#2 |  |  |  |
| AA35 | VTT4 | AH6 | A10# |  |  |  |
| AA37 | VccCORE | AH8 | A5# |  |  |  |
| AB2 | VccCORE | AH10 | A8# |  |  |  |
| AB4 | A24# | AH12 | A4# |  |  |  |
| AB6 | A23# | AH14 | BNR# |  |  |  |
| AB32 | GND | AH16 | REQ1# |  |  |  |
| AB34 | VccCORE | AH18 | REQ2# |  |  |  |
| AB36 | VccCMOS | AH20 | VTT |  |  |  |
| AC1 | A33# | AH22 | RS1# |  |  |  |
| AC3 | A20# | AH24 | VccCORE |  |  |  |
| AC5 | GND | AH26 | RS0# |  |  |  |
| AC33 | GND | AH28 | THERMTRIP# |  |  |  |
| AC35 | FERR# | AH30 | SLP# |  |  |  |
| AC37 | RSP# | AH32 | VccCORE |  |  |  |
| AD2 | GND | AH34 | GND |  |  |  |
| AD4 | A31# | AH36 | VccCORE |  |  |  |
| AD6 | VREF5 | AJ1 | A21# |  |  |  |
| AD32 | VccCORE | AJ3 | GND |  |  |  |
| AJ5 | VccCORE | AL11 | AP0# |  |  |  |
| AJ7 | GND | AL13 | VTT |  |  |  |
| AJ9 | VccCORE | AL15 | A7# |  |  |  |
| AJ11 | GND | AL17 | REQ4# |  |  |  |
| AJ13 | VccCORE | AL19 | REQ3# |  |  |  |
| J15 | GND | AL21 | VTT |  |  |  |
| AJ17 | VccCORE | AL23 | HITM# |  |  |  |
| AJ19 | GND | AL25 | HIT# |  |  |  |
| AJ21 | VccCORE | AL27 | DBSY# |  |  |  |
| AJ23 | GND | AL29 | THERMDN |  |  |  |
| AJ25 | VccCORE | AL31 | THERMDP |  |  |  |
| AJ27 | GND | AL33 | TCK |  |  |  |
| AJ29 | VccCORE | AL35 | VID0 |  |  |  |
| AJ31 | BSEL1 | AL37 | VID2 |  |  |  |
| AJ33 | BSEL0 | AM26 | Reserved |  |  |  |
| AJ35 | SMI# | AM4 | VccCORE |  |  |  |
| AJ37 | VID3 | AM6 | GND |  |  |  |
| AK2 | VccCORE | AM8 | VccCORE |  |  |  |
| AK4 | GND | AM10 | GND |  |  |  |
| AK6 | A28# | AM12 | VccCORE |  |  |  |
| AK8 | A3# | AM14 | GND |  |  |  |
| AK10 | A11# | AM16 | VccCORE |  |  |  |
| AK12 | VREF6 | AM18 | GND |  |  |  |
| AK14 | A14# | AM20 | VccCORE |  |  |  |
| AK16 | VTT | AM22 | GND |  |  |  |
| AK18 | REQ0# | AM24 | VccCORE |  |  |  |
| AK20 | LOCK# | AM26 | GND |  |  |  |
| AK22 | VREF7# | AM28 | VccCORE |  |  |  |
| AK24 | AERR# | AM30 | GND |  |  |  |
| AK26 | PWRGOOD | AM32 | VccCORE |  |  |  |
| AK28 | RS2# | AM34 | GND |  |  |  |
| AK30 | Reserved | AM36 | VID1 |  |  |  |
| AK32 | TMS | AN3 | GND |  |  |  |
| AK34 | VccCORE | AN5 | A12# |  |  |  |
| AK36 | GND | AN7 | A16# |  |  |  |
| AL1 | GND | AN9 | A6# |  |  |  |
| AL3 | GND | AN11 | VTT |  |  |  |
| AL5 | A15# | AN13 | AP1# |  |  |  |
| AL7 | A13# | AN15 | VTT |  |  |  |
| AL9 | A9# | AN17 | BPRI# |  |  |  |
| AN19 | DEFER# | C25 | D50# |  |  |  |
| AN21 | VTT4 | C27 | D56# |  |  |  |
| AN23 | RP# | C29 | DEP5# |  |  |  |
| AN25 | TRDY# | C31 | DEP1# |  |  |  |
| AN27 | DRDY# | C33 | DEP0# |  |  |  |
| AN29 | BR0# | C35 | BPM0# |  |  |  |
| AN31 | ADS# | C37 | CPUPRES# |  |  |  |
| AN33 | TRST# | D2 | GND |  |  |  |
| AN35 | TDI | D4 | GND |  |  |  |
| AN37 | TDO | D6 | VccCORE |  |  |  |
| B2 | D35# | D8 | D38# |  |  |  |
| B4 | GND | D10 | D39# |  |  |  |
| B6 | VccCORE | D12 | D42# |  |  |  |
| B8 | GND | D14 | D41# |  |  |  |
| B10 | VccCORE | D16 | D52# |  |  |  |
| B12 | GND | D18 | GND |  |  |  |
| B14 | VccCORE | D20 | VccCORE |  |  |  |
| B16 | GND | D22 | GND |  |  |  |
| B18 | VccCORE | D24 | VccCORE |  |  |  |
| B20 | GND | D26 | GND |  |  |  |
| B22 | VccCORE | D28 | VccCORE |  |  |  |
| B24 | GND | D30 | GND |  |  |  |
| B26 | VccCORE | D32 | VccCORE |  |  |  |
| B28 | GND | D34 | GND |  |  |  |
| B30 | VccCORE | D36 | VccCORE |  |  |  |
| B32 | GND | E1 | D26# |  |  |  |
| B34 | VccCORE | E3 | D25# |  |  |  |
| B36 | BINIT# | E5 | VccCORE |  |  |  |
| C1 | D33# | E7 | GND |  |  |  |
| C3 | VccCORE | E9 | VccCORE |  |  |  |
| C5 | D31# | E11 | GND |  |  |  |
| C7 | D34# | E13 | VccCORE |  |  |  |
| C9 | D36# | E15 | GND |  |  |  |
| C11 | D45# | E17 | VccCORE |  |  |  |
| C13 | D49# | E19 | GND |  |  |  |
| C15 | D40# | E21 | RESERVE |  |  |  |
| C17 | D59# | E23 | VTT4 |  |  |  |
| C19 | D55# | E25 | D62# |  |  |  |
| C21 | D54# | E27 | SLEWCTRL |  |  |  |
| C23 | D58# | E29 | DEP6# |  |  |  |
| E31 | DEP4# | K2 | VccCORE |  |  |  |
| E33 | VREF0 | K4 | VREF2 |  |  |  |
| E35 | BPM1# | K6 | D24# |  |  |  |
| E37 | BP3# | K32 | VccCORE |  | AH12 | A4# |
| F2 | VccCORE | K34 | VccCORE |  |  |  |
| F4 | VccCORE | K36 | GND |  |  |  |
| F6 | D32# | L1 | D13# |  |  |  |
| F8 | D22# | L3 | D20# |  |  |  |
| F10 | Reserved | L5 | GND |  |  |  |
| F12 | D27# | L33 | Reserved |  |  |  |
| F14 | VccCORE | L35 | PICD1 |  |  |  |
| F16 | D63# | L37 | LINT1/NMI |  |  |  |
| F18 | VREF1 | M2 | GND |  |  |  |
| F20 | GND | M4 | D11# |  |  |  |
| F22 | VccCORE | M6 | D3# |  |  |  |
| F24 | GND | M32 | VccCORE |  |  |  |
| F26 | VccCORE | M34 | GND |  |  |  |
| F28 | GND | M36 | LINT0/INTR |  |  |  |
| F30 | VccCORE | N1 | D2# |  |  |  |
| F32 | GND | N3 | D14# |  |  |  |
| F34 | VccCORE | N5 | VccCORE |  |  |  |
| F36 | GND | N33 | Reserved |  |  |  |
| G1 | D21# | N35 | Reserved |  |  |  |
| G3 | D23# | N37 | Reserved |  |  |  |
| G5 | GND | P2 | VccCORE |  |  |  |
| G33 | BP2# | P4 | D18# |  |  |  |
| G35 | VTT | P6 | D9# |  |  |  |
| G37 | Reserved | P32 | GND |  |  |  |
| H2 | GND | P34 | VccCORE |  |  |  |
| H4 | D16# | P36 | GND |  |  |  |
| H6 | D19# | Q1 | D12# |  |  |  |
| H32 | VccCORE | Q3 | D10# |  |  |  |
| H34 | GND | Q5 | GND |  |  |  |
| H36 | VccCORE | Q33 | Reserved |  |  |  |
| J1 | D7# | Q35 | Reserved |  |  |  |
| J3 | D30# | Q37 | Reserved |  |  |  |
| J5 | VccCORE | R2 | Reserved |  |  |  |
| J33 | PICCLK | R4 | D17# |  |  |  |
| J35 | PICD0 | R6 | VREF3 |  |  |  |
| J37 | PREQ# | R32 | VccCORE |  |  |  |
| R34 | GND | V36 | VccCORE |  |  |  |
| R36 | VccCORE | W1 | D0# |  |  |  |
| S1 | D8# | W3 | A34# |  |  |  |
| S3 | D5# | W5 | VccCORE |  |  |  |
| S5 | VccCORE | W33 | PLL1 |  |  |  |
| S33 | VTT4 | W35 | Reserved |  |  |  |
| S35 | RTTCTRL | W37 | BCLK |  |  |  |
| S37 | VTT4 | X2 | BR1# |  |  |  |
| T2 | VccCORE | X4 | RESET2#2 |  |  |  |
| T4 | D1# | X6 | A32# |  |  |  |
| T6 | D6# | X32 | GND |  |  |  |
| T32 | GND | X34 | VccCORE |  |  |  |
| T34 | VccCORE | X36 | GND |  |  |  |
| T36 | GND | Y1 | Reserved |  |  |  |
| U1 | D4# | Y3 | A26# |  |  |  |
| U3 | D15# | Y5 | GND |  |  |  |
| U5 | GND | Y33 | CLKREF7 |  |  |  |
| U33 | PLL2 | Y35 | VccCORE |  |  |  |
| U35 | VTT4 | Y37 | GND |  |  |  |
| U37 | VTT4 | Z2 | GND |  |  |  |
| V2 | GND | Z4 | A29# |  |  |  |
| V4 | BERR# | Z6 | A18# |  |  |  |
| V6 | VREF4 | Z32 | VccCORE |  |  |  |
| V32 | VCCCORE | Z34 | GND |  |  |  |
| V34 | GND | Z36 | VCC2.51 |  |  |  |

### JP21 FAN.PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | FAN_SENSE | 2. | POWER |
| 3. | GND |  |  |

### JP7 FDD.PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | +5VS | 2. | INDEX# |
| 3. | +5VS | 4. | DRV0# |
| 5. | +5VS | 6. | DSKCHG# |
| 7. | NC | 8. | NC |
| 9. | NC | 10. | MTR0# |
| 11. | NC | 12. | FDDIR# |
| 13. | 3MODE# | 14. | STEP# |
| 15. | NC | 16. | NC |
| 17. | GND | 18. | WGARE# |
| 19. | GND | 20. | TRACK0# |
| 21. | GND | 22. | WP# |
| 23. | GND | 24. | RDATA# |
| 25. | GND | 26 | HDSEL# |

### PJP2 ACIN PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | VIN | 2. | GND |
| 3. | GND | 4. |  |

### SW1 LID-SW

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | GND | 2. | GND |
| 3. | LID_SW# | 4. | LID_SW# |

### PJP3 BATT-A.PRT(FOR 38W2 ONLY)

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | VMBA | 2. | ALI/NIMH# |
| 3. | AB/I | 4. | TSA |
| 5. | N20122 | 6. | N20183 |
| 7. | GND |  |  |

### PCN1 DC JACK.PRT (30W ONLY)

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | DC_IN | 2. | GND_DCJACK |
| 3. | GND |  |  |

### PCN2 DC JACK.PRT (38W ONLY)

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | DC_IN | 2. | GND_DCJACK |
| 3. | GND |  |  |

### J1 LINEIN.PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | GNDA | 2. | LINE_IN_L |
| 3. | LINE_IN_R | 4. | NC |
| 5. | NC | 6 | NC |

### J2 EXT_MIC.PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | GNDA | 2. | EXT_MIC |
| 3. | NC | 4. | EXT_MIC_PLUG# |
| 5. | N2333 | 6. | DOCK_MIC |

### J3 LINE_OUT.PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | AGND | 2. | LINEOUTL |
| 3. | LINEOUTR | 4. | LINE_OUT_PLUG |
| 5. | GND | 6. | NC |

### JP14 CD.PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | INT_CD_L | 2. | INT_CD_R |
| 3. | CD_AGND | 4. | GND |
| 5. | SIDERST# | 6. | CDD8 |
| 7 | CDD7 | 8. | CDD9 |
| 9. | CDD6 | 10. | CDD10 |
| 11. | CDD5 | 12. | CDD11 |
| 13. | CDD4 | 14. | CDD12 |
| 15. | CDD3 | 16. | CDD13 |
| 17. | CDD2 | 18. | CDD14 |
| 19. | CDD1 | 20. | CDD15 |
| 21. | CDD0 | 22. | CD-DREQ |
| 23. | GND | 24. | CD_SIOR# |
| 25. | CD_SIOW | 26. | GND |
| 27. | CD_SIORY | 28. | CD_DACK# |
| 29. | CD_IRQ | 30. | NC |
| 31. | CD_SBA1 | 32. | PDIAG# |
| 33. | CD_SBA0 | 34. | CD_SBA2 |
| 35. | CD_SCS1# | 36. | CD_SCS3# |
| 37. | SHDD_LED# | 38. | +5VCD |
| 39. | +50CD | 40. | +50CD |
| 41. | +50CD | 42. | +50CD |
| 43. | GND | 44. | GND |
| 45. | GND | 46. | GND |
| 47. | NC | 48. | GND |
| 49. | NC | 50. | NC |

### JP19 PCMCIA-CONN. 154P.PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| A1. | GND | B1 | GND |
| A2. | GND | B2 | GND |
| A3. | S1_D3 | B3 | S2_D3 |
| A4. | S1_CD1# | B4 | S2_CD1# |
| A5 | S1 D4 | B5 | S2 D4 |
| A6 | S1 D11 | B6 | S2 D11 |
| A7 | S1 D5 | B7 | S2 D5 |
| A8 | S1_D12 | B8 | S2_D12 |
| A9 | GND | B9 | GND |
| A10 | S1 D6 | B10 | S2 D6 |
| A11 | S1 D13 | B11 | S2 D13 |
| A12 | S1 D7 | B12 | S2 D7 |
| A13 | D14 | B13 | S2 D14 |
| A14 | S1 CE1# | B14 | S2 CE1# |
| A15 | S1 D15 | B15 | S2 D15 |
| A16 | GND | B16 | GND |
| A17 | S1 A10 | B17 | S2 A10 |
| A18 | S1 CE2# | B18 | S2 CE2# |
| A19 | S1 OE# | B19 | S2 OE# |
| A20 | S1 VS1 | B20 | S2 VS1 |
| A21 | S1 A11 | B21 | S2 A11 |
| A22 | GND | B22 | GND |
| A23 | S1 10RD# | B23 | S2 10RD# |
| A24 | S1 A9 | B24 | S2 A9 |
| A25 | S1 10WR# | B25 | S2 10WR# |
| A26 | S1 A8 | B26 | S2 A8 |
| A27 | S1 A17 | B27 | S2 A17 |
| A28 | GND | B28 | GND |
| A29 | S1 A13 | B29 | S2 A13 |
| A30 | S1_A18 | B30 | S2_A18 |
| A31 | S1 A14 | B31 | S2 A14 |
| A32 | S1 A19 | B32 | S2 A19 |
| A33 | S1 WE# | B33 | S2 WE# |
| A34 | S1 A20 | B34 | S2 A20 |
| A35 | S1 RDY# | B35 | S2 RDY# |
| A36 | S1 A21 | B36 | S2 A21 |
| A37 | S1_VCC | B37 | S2_VCC |
| A38 | S1_VCC | B38 | S2_VCC |
| A39 | S1_VPP | B39 | S2_VPP |
| A40 | S1_VPP | B40 | S2_VPP |
| A41 | S1 A16 | B41 | S2 A16 |
| A42 | GND | B42 | GND |
| A43 | S1 A22 | B43 | S2 A22 |
| A44 | S1 A15 | B44 | S2 A15 |
| A45 | S1 A23 | B45 | S2 A23 |
| A46 | S1 A12 | B46 | S2 A12 |
| A47 | S1 A24 | B47 | S2 A24 |
| A48 | S1 A7 | B48 | S2 A7 |
| A49 | GND | B49 | GND |
| A50 | S1 A25 | B50 | S2 A25 |
| A51 | S1_A6 | B51 | S2_A6 |
| A52 | S1 VS2 | B52 | S2 VS2 |
| A53 | S1 A5 | B53 | S2 A5 |
| A54 | S1 RST | B54 | S2 RST |
| A55 | S1 A4 | B55 | S2 A4 |
| A56 | S1 WAIT# | B56 | S2 WAIT# |
| A57 | GND | B57 | GND |
| A58 | S1 A3 | B58 | S2 A3 |
| A59 | S1 INPACK# | B59 | S2 INPACK# |
| A60 | S1 A2 | B60 | S2 A2 |
| A61 | S1 REG# | B61 | S2 REG# |
| A62 | S1 A1 | B62 | S2 A1 |
| A63 | S1 BVD2 | B63 | S2 BVD2 |
| A64 | S1 A0 | B64 | S2 A0 |
| A65 | GND | B65 | GND |
| A66 | S1 BVD1 | B66 | S2 BVD1 |
| A67 | S1 D0 | B67 | S2 D0 |
| A68 | S1 D8 | B68 | S2 D8 |
| A69 | S1 D1 | B69 | S2 D1 |
| A70 | S1 D9 | B70 | S2 D9 |
| A71 | S1 D2 | B71 | S2 D2 |
| A72 | S1 D10 | B72 | S2 D10 |
| A73 | GND | B73 | GND |
| A74 | S1 WP | B74 | S2 WP |
| A75 | S1 CD2# | B75 | S2 CD2# |
| A76 | GND | B76 | GND |
| A77 | GND | B77 | GND |

### JP4 HEADERX50

| PIN NO | SIGNAL | PIN NO | SIGNAL |  |
| --- | --- | --- | --- | --- |
| 1. | NC | 2. | BTN1# |  |
| 3. | BTN2# | 4. | MAIL_ACT_LED# |  |
| 5. | BTN4# | 6. | BTN3# |  |
| 7. | CDON_BTN# | 8. | VOL_UP# |  |
| 9. | VOL_DW# | 10. | VOL_AMP |  |
| 11. | NC | 12. | PWR_LED# |  |
| 13. | BUT LOCK# | 14. | EMAIL ON# |  |
| 15. | PS2_CLK | 16. | PS2_DATA |  |
| 17. | NC | 18. | NC |  |
| 19. | NC | 20. | NC |  |
| 21. | NC | 22. | IAC_BITCLK |  |
| 23. | IAC_SDATAI | 24. | IAC_SDATAO |  |
| 25. | IAC_SYNC | 26. | IAC_RST# |  |
| 27. | GND | 28. | BATT_LOW_LED# |  |
| 29. | GND | 30. | DM_ON |  |
| 31. | DM_ON# | 32. | GND |  |
| 33. | BATT_CHGI_LED# | 34. | NC |  |
| 35. | MD_SPK | 36. | NC |  |
| 37. | MONO IN R | 38. | CD_AGND |  |
| 39. | CD_AGND | 40. | CDROM_L |  |
| 41. | CD_AGND | 42. | +3VS |  |
| 43. | NC | 44. | +5VCD |  |
| 45. | +5VCD | 46. | +5VCD |  |
| 47. | NC | 48. | +5VAW |  |
| 49. | +5VS | 50. | +5VS |  |
| ### P2 LCD |  |  |  |  |
| PIN NO | SIGNAL | PIN NO |  | SIGNAL |
| 1. | +5VALW | 2. |  | GND |
| 3. | +5VALW | 4. |  | GND |
| 5. | NC | 6. |  | NC |
| 7. | 20 INVT_PWN | 8. |  | DISPOFF# |
| 9. | +5VALW | 10. |  | LCDDVDD |
| 11. | +5VALW | 12. |  | LCDDVDD |
| 13. | +3VS | 14. |  | LCDDVGA |
| 15. | +3VS | 16. |  | LCDDVGA |
| 17. | GND | 18. |  | GND |
| 19. | NC | 20. |  | L LCD0 |
| 21. | GND | 22. |  | L LCD1 |
| 23. | PID0 | 24. |  | GND |
| 25. | PID1 | 26. |  | L LCD2 |
| 27. | PID2 | 28. |  | L LCD3 |
| 29. | PID3 | 30. |  | GND |
| 31. | ENVDD | 32. |  | L LCD4 |
| 33. | DISPOFF# | 34. |  | L LCD5 |
| 35. | L LCD15 | 36. |  | GND |
| 37. | L LCD14 | 38. |  | L LCD6 |
| 39. | GND | 40. |  | L LCD7 |
| 41. | L LCD13 | 42. |  | GND |
| 43. | L LCD12 | 44. |  | L LCD8 |
| 45. | GND | 46. |  | L LCD9 |
| 47. | L LCD11 | 48. |  | GND |
| 49. | L LCD10 | 50. |  | NC |

### ZP14 CD.PRT

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | INT_CD_L | 2. | INT_CD_R |
| 3. | CD_AGND | 4. | GND |
| 5. | SIDERST# | 6. | CDD8 |
| 7 | CDD7 | 8. | CDD9 |
| 9. | CDD6 | 10. | CDD10 |
| 11. | CDD5 | 12. | CDD11 |
| 13. | CDD4 | 14. | CDD12 |
| 15. | CDD3 | 16. | CDD13 |
| 17. | CDD2 | 18. | CDD14 |
| 19. | CDD1 | 20. | CDD15 |
| 21. | CDD0 | 22. | CD-DREQ |
| 23. | GND | 24. | CD_SIOR# |
| 25. | CD_SIOW | 26. | GND |
| 27. | CD_SIORY | 28. | CD_DACK# |
| 29. | CD_IRQ | 30. | NC |
| 31. | CD_SBA1 | 32. | PDIAG# |
| 33. | CD_SBA0 | 34. | CD_SBA2 |
| 35. | CD_SCS1# | 36. | CD_SCS3# |
| 37. | SHDD_LED# | 38. | +5VCD |
| 39. | +50CD | 40. | +5VCD |
| 41. | +50CD | 42. | +5VCD |
| 43. | GND | 44. | GND |
| 45. | GND | 46. | GND |
| 47. | NC | 48. | GND |
| 49. | NC | 50. | NC |

### JP6  KBD

| PIN NO | SIGNAL | PIN NO | SIGNAL |
| --- | --- | --- | --- |
| 1. | KSO15 | 2. | KSO10 |
| 3. | KSO11 | 4. | KSO14 |
| 5. | KSO13 | 6. | KSO12 |
| 7. | KSO3 | 8. | KSO6 |
| 9. | KS08 | 10. | KSO7 |
| 11. | KS04 | 12. | KSO2 |
| 13. | KSI0 | 14. | KSO1 |
| 15. | KSO5 | 16. | KSI3 |
| 17. | KSI2 | 18. | KSO0 |
| 19. | KS15 | 20. | KSI4 |
| 21. | KSO9 | 22. | KSO9 |
| 23. | KSI7 | 24. | KSI1 |