Chapter 1
System Description
Specification

# SCOPE

This document describes the functional specifications for the Compal NoteBook personal computer HEL81 series. The system is hardware and software compatible with the IBM PC/AT personal computer.

## CPU

- Intel Pentium M Yonah mobile dual core processor FSB 667MHz, 2MB L2 T2300(1.66GHz)/T2400(1.83GHz)/T2500(2GHz)/T2600(2.16GHz) with 31W TDP

- Intel Pentium M Yonah mobile single core processor ,FSB 667MHz (Design ready)
T1300(1.66GHz)/T1400(1.83GHz) with 31W TDP

- Intel MEROM design ready

## Chip Set

- Intel 945GM + ICH7-M (940GML design ready for entry SKU with another EN schedule)

- 533/667 MHz FSB support

## Memory

- No on board memory.

- Support DDR-II 533/667MHz

- Two SODIMM with 512MB/1GB modules

- Easy upgrade from bottom side for two SODIMM. Max. 2GB

## BIOS

- Phoenix First BIOS

- 1024KB Flash BIOS ROM

- Suspend to RAM/Disk

- No support APM

- Security : Power-On Password, Supervisor Password, HDD password 
USB memory Boot support

- Support SMBIOS 2.3, PCI2.2.

- ACPI 2.0 compliance with Intel Speed-step Support C1, C2, C3, C4 and S3,  
S4 for mobile CPU

- DMI utility for BIOS serial number configurable/asset tag

- Support PXE

- Wake on LAN from S3

- Wake on LAN form S4 in AC mode

## Power

- 6-cell LI-ION of 18650 size battery pack .( 10.8V x 4800mAh)

- 9 cell LI-ION of 18650 size battery pack (10.8V x 7200mAh)

- Support quick charging, 70%~80% in around 1hr.

- Life Cycle: 70% Design Capacity after 300 Cycles in 25degreeC

- Battery life: (Tested by Mobilemark2002, v1.0 patch2 by battery life only

- 3.6H(typical) with 9 cell for Ext. Gfx + Pentium-M

## HDD

- 9.5mm, 2.5" P-ATA HDD support with 40/60/80/100GB ( 5400rpm ) capacity

- Easily removable no more than two screws

## ODD

- Fixed 12.7mm height

- Combo(CD-RW/DVD-ROM) (24x/24x/8x/24x)

- DVD-RAM/-R/-RW readable

- DVD Super-Multi (DVD-RAM/-R/-RW/+R/+RW)

- 8X write

- Support Double-Layer Recording is required

## Options

- 64MB/128/256MB SO-DIMM modules

## Touch Pad with two buttons

- Left Button

- Right Button

## I/O Ports

- No port Cover

- USB (V2.0, horizontal type) x 3 (2 in Right side, 1 in Left side)

- RGB; VGA port x 1, 15pins(Rright side)

- S-Video out x 1(Rear side)

- Microphone-in x 1(Right side)

- Headphone-out jack with S/PDIF x 1(Right side)

- No FIR.

- DC-in jack(Rear side)

- RJ-11 jack x 1 for 56Kbps V.90/92 Modem(Rear side)

- RJ-45 jack x 1 (Left side)

- Kensington lock x 1(Left side)

- Express Card/54 x 1(Left side)

- PCMCIA Type II x 1 (Left side)

- 3-in-1 Flash Card Reader(SD/MMC/MS) x 1(front side)

- IEEE 1394 x1(4-pin) x 1(left side)

- Two way communication switch(Left to off, Right to on)

- With CIR

- Internal microphone x 1(in LCD module)

## PCMCIA Card Organization

- ENE CB714 controller

- PC card 95 supported with 1x Type II

- PCI – Express card (54mm type) x1ACPI 1.0 Compliant

## AC Adapter

- Universal AC adapter. 90-264V AC, 47-63HZ

- 90W with 19V DC

- 3 pin 240V AC cable .

## Security

- HDD Password

- TPM 1.2 by Infineon

- Design ready for Windows Vista

- Fingerprint

- Kensington lock

## Memory Card Reader

- 3-in-1 Card Reader (MS, SD, MMC)

## Display

- In Support 15.4” WXGA, anti-glare and anti-reflection

## Graphics Chip

- UMA (Embedded in Intel 945GM

## Audio

- No on board memory.

- Realtek ALC883

- HD Audio

- Microphone-in and headphone-out

- Internal Microphone

- Two stereo speakers

- Microphone array supported

- Volume control: Hot-key control for XP

## Communication

- A. Modem	 B. LAN  C. WLAN  D. Bluetooth(BTO)  E. Camera Wake on LAN form S4 in AC mod

- Internal Modem with MDC 1.5 solution

- V.92/56K bps; V.90/56K bps

- LAN: Realtek 8111B for Giga bit LAN

- Wireless LAN, 802.11b/g, a/b/g via Mini-Card interface

- Bluetooth Ver. 2.0 module with USB2.0 interface

## Keyboard

- 85/86 keys support with 101/102 key emulation without stick-point.

- Windows key, Application key

- Standard pitch, 3.0mm travel length.

- Multi-Language support

## IEEE 1394

- VIA VT6311S.

- PCI single chip solution, PCI 2.2 compliant.

- OHCI v1.1

- IEEE1394a  4-pin

## System Status Indicators

- 3 K/B indicators: Caps Lock (White), and Num Lock (White), Scroll Lock (White)

- 1 Power Button LED (Blue)

- 1 LED for IDE HDD activity (Green)

- 1 LED for System status (Power On / Suspend) (Blue)

- 1 LED for Battery status (Charging / Full / L1 / L2) (Green/Amber)

- 1 LED for Wireless Card (RF ON/OFF) (Amder)

- 1 3-in-1 card reader(Green)

## Control

- Power button x 1

- Magnetic lid switch control for system standby/wakeup or suspend/resume

- One Switch to On/Off Wireless Device for wireless control

## User Keys

- Control button for short-Cut key functions

- 1 Internet (Explorer) key

- 1 E-mail (Outlook) key

## OS

- Windows XP HE(SP2),

- Windows XP Pro(SP2)(with PC 2001 LOGO, and K/B LOGO)

- Windows Vista design ready

## Mini Card

- One Mini card slot for WLAN

- One Mini card slot for Hybrid TV

## Other requirement

- TV tuner module with Cable

## Regulatory

- EMI: FCC-B, CE, CCC, A,C-Tick, VCCI, BSMI

- Safety: Compliant to UL/CSA, TUV, CB

- PTT:  FCC-Part 68, CTR 21

- MDA2005 Compliant

- Energy Star

- WHQL LOGO

## Environment

- Temperature:

- Operating: 5 - 35°C

- Storage: -20 - 65°C

- Humidity:

- 10 - 90% without condensation.

- Altitude:

- Operating sea level up to 10,000 ft

- Storage sea level up to 40,000 ft

- Intel MEROM design ready

## Thermal(TBD)

- No throttling occur with running SysMark2004 / 3DMark03/3DMark05

- Business Winston 2004/ MCC Winston 2004 / PCMark04 Pro at 35oC

- Thermal protection

- HW thermal protection:

- Shutdown system ( Thermister for CPU ) : 84°C +/-3°C (local

- temperature of　PCB)

- Shutdown system ( Thermister for Battery ) : 79°C +/-3°C (local

- temperature of PCB)

- CPU thermal protection

- Throttling : TCC enabled

- Shutdown : 125°C (T-junction of CPU)

- Watchdog protection.

- SW shutdown: 100°C(Tj)

- Acoustic noise

- 42 dBA for Dothan CPU @35C

## Dimension

- 362mm(W) x 265mm(D)x 31.7mm(H) in front side, 38.9mm in rear side

## Weight

- Less than 2.73 kg with 15.4”WXGA & 6cell battery