Chapter 2
Software Specification

## Software Specification for System BIOS

## System Controls

### HotKeys Support

Listing below are the hot keys supported:

| Hotkey | Area | Feature | Software involved | Note |
| --- | --- | --- | --- | --- |
| F2 | Preboot (POST) | Enters BIOS Setup | BIOS (POST) |  |
| Fn+F1 | OS | Enters S3 sleep state |  | Either “Sleep Button” defined in ACPI specification Or handled by OEM PM Driver |
| Fn+F2 | OS | Wirelss Enable/Disable |  |  |
| Fn+F3 | OS | LCD/CRT/LCD+CRT Switch |  |  |
| Fn+F4 | OS | Brightness down |  |  |
| Fn+F5 | OS | Brightness up |  |  |
| Fn+F6 | OS | Mute |  |  |
| Fn+F7 | OS | Volume Down |  |  |
| Fn+F8 | OS | Volume Up |  |  |
| Fn+F9 | OS | Media Play/Pause |  |  |
| Fn+F10 | OS | Media Stop |  |  |
| Fn+F11 | OS | Media Play Backward |  |  |
| Fn+F12 | OS | Media Play Forward |  |  |
| Fn+Esc | OS | BT enable/disable |  |  |

### Buttons

#### Power Button

The activity of the power button is as follows:

If system is Off/Hibernate: System will be turned on while Power switch is depressed by more than 100 ms

If system is in Standby state: System will resume while Power switch is depressed by more than 100 ms.

If system on with legacy mode: depress this button will turn off power.

If system is running in ACPI OS, the power button acts as the sleep button, and let OS controls the policy of power button which is defined in Power Option under the OS.

#### Power Button Over-ride

- Holding down the Power Button for 4 seconds will cause an unconditional transfer to the off state without notifying the operating system.

#### Lid Switch

- This section describes the expected behavior of the system when the lid is opened or closed by the user.

- If the system is running under legacy mode:

- Closing the lid will turn off LCD backlight.

If the system is running under ACPI mode:

- The operating system will determine what action to take when the lid is opened and closed.

- The function of lid switch will follow the OS setting in power management (Nothing, Standby or Hibernate). If nothing, the backlight must turn off when the lid is closed.

#### System status indicators

Please refer to Keyboard BIOS specification.

## Core BIOS Features

### Boot

According to Microsoft Simple Boot Flag Specification v1.0, the OEM model default should have Fast boot implemented. Hence, there is no menu option for Fast Boot enable/disable in the BIOS setup menu.

### Multi Boot

The notebook can support Multi Boot for selecting the boot sequence of hard disk, floppy, CD ROM, Network Boot in Setup. It identifies all IPL (Initial Program Load) devices in the system and attempts to boot them in the order specified in Setup.

### Quiet Boot

Quiet Boot replaces the customary technical messages during POST with a more visually pleasing and comfortable display (OEM Logo screen). BIOS must support the customized BIOS (POST) splash screen.

During POST, right after the initialization of VGA, The notebook displays an illustration called the OEM screen during system boot instead of the traditional POST screen that displays the normal diagnostic messages.

The OEM Logo screen stays up until just before the operating system loads unless:

- Press <F2> to enter Setup.

(When pressed, need to show “Entering Setup…”)

- Press <F12> to change the boot order.

(When pressed, need to shows “Press [F1] for SETUP, Entering boot device selection menu …”

- Whenever POST detects a non-terminal error, it switches to the POST screen near the end of POST, just prior to prompting for a password.

- If the BIOS or an option ROM requests keyboard input, the system switches over to the POST screen with prompts for entering the information. POST continues from there with the regular POST screen.

### Enhanced IDE Disk Drive Support (EDD)

In addition to AT standard disk drive support, the Phoenix NoteBIOS 4.06 also supports:

·	Auto-detection and sizing of all IDE drives.

·	Logical Block Addressing(LBA)

·	Fast DMA support

·	Ultra DMA-33/66/100/133 support

### New Interrupt 15h extensions

The BIOS must support the recently defined standard INT 15 extensions.

Big memory support that can reporting greater than 64 megabytes of RAM.  The notebook supports the INT 15h big-memory reporting functions of E801h, E881h, and E820h.  This feature reports all available extended memory (both below and above the 64MB limit) using both a real mode (E801h) and a 32-bit protected mode (E881h) interface. Operating systems can access the real-mode interface through the standard INT 15h call.  They can access the protected-mode interface through a 32-bit interrupt call, much like the EISA protected-mode interface. The Microsoft-defined E820h function returns a complete memory map through a series of repeated calls.

### Boot Block

- The Flash ROM used in many systems today offer the customer the advantage of electronically reprogramming the BIOS without physically replacing the BIOS ROM. This advantage, however, does create a possible hazard: power failures or fluctuations that occur during updating the Flash ROM can damage the BIOS code, making the system unbootable.  To prevent this possible hazard, many Flash ROM include a special non-volatile region that can never be erased. This region, called the boot block, contains a fail-safe recovery routine. If the boot block finds corrupted BIOS, it prompts the end user to insert a diskette, from which it loads several files that replace the corrupted BIOS on the Flash ROM with an uncorrupted one.

### Plug-n-Play (PnP) Support

To achieve the goal of PnP, a POST conflict detection and resolution (CDR) module, and a run-time services module will be integrated into the system BIOS.

The PnP runtime service module includes multiple interfaces so that the system can support the current DOS/Win3.1 non-PnP drivers, as well as Win98 operating system that include specific support for the PnP BIOS specification.

## Security Features

Security features to be supported are passwords, electronic serial number, PC identification string.

The Electronic Serial Number provides a unique way of identifying an individual notebook.

PC Identification strings allow the user or Supervisor to personalize the notebook for asset tracking or identification if it is lost.

### 2 Level Passwords

The notebook supports two levels of password protection. The password support consists of a User Password and an Supervisor Password.  They each contain up to eight characters, and are stored in a non-volatile storage device(CMOS). Using the administration password to enter the setup utility allows the user to access all the configurable fields. Whereas using the user password only allows the user to configure a limited number of fields.

When the password is enabled, the notebook may display a suitable password prompt on the main display in the following situations:.

- Turning on from off states. (No BIOS suspend/resume password)

- Entering to Setup.

The User will attempt to enter a password, then press ENTER.  If the User fails to enter the password in three tries the system will be halt.

#### User Password

The user can choose:

- The password will never be required

- Be required to boot.

- The user password may not be set unless the Supervisor password is set.  If the user wishes to only have one password then the Supervisor password is used.

- The User Password can change the following items in the BIOS:

Main:

- System Date

- System Time

System Devices:

- Quiet Boot

- Boot Display Device

- Internal TouchPad

- Internal Modem

Security:

- Set User Password

- The Supervisor Password can change all the settings in the BIOS

#### Supervisor Password

There are two primary uses for the Supervisor Password:

- Protect users from changing system configuration that could cause the notebook to malfunction.

- As the users password if only a single password is desired.

#### Passwords and Setup

There are individual checkbox items in Setup to allow the user to specify when to require a password.  The wording of these options should be:

Password Required to:

Boot				[Enabled]

The following table lists the items in the Setup utility which can be modified by the User.

Time & Date

User password

While setting new password, three failures to enter the old password will result in the system turning off.

#### Valid Password Characters

Valid Password Characters

- The numbers 0 to 9.

- The letters A to Z (not case sensitive).

- Both passwords will be encrypted before being stored in CMOS RAM using the standard Phoenix password encryption technique.

#### Secured Data

Secured data is kept in a non-volatile storage device (Inverter EEPROM) and is secured with a write protection control. The entire data structure of secured data in the non-volatile storage must be verified with a checksum calculation. This information will be displayed on the SETUP screen.

Data structure redundancy may be employed to guard against storage device bit failures.

### Unit Serial Number

Every unit will contain its Serial number in the secured data area. The Serial Number is an alphanumeric string of 20 bytes in length with checksum.

### Product Name

A Product Name string will be stored in the secured data area. The product name is an alphanumeric string of 16 bytes in length with checksum

### Manufacturer Name

A Manufacturer Name string will be stored in the secured data area. The Manufacturer Name is an alphanumeric string of 16 bytes in length with checksum.

### UUID

A UUID string will be stored in the secured data area. The UUID is also need to be displayed on SETUP menu in this product.

The following is an example of the string representation of a UUID:

a31e4fae8dbc81d2a11590a8e91e6cf6

For PC2001 BIOS-0014.3 requirement, UUID should print on the system chassis or case, or on the shipping carton.

## Thermal management

There are two types of cooling techniques used for thermal management.  The first type is passive cooling where the CPU speed is reduced or other devices power consumption is reduced in an effort to reduce heat generation.  The second type is active cooling where a fan is turned on to cool the system.

In non-ACPI mode, the thermal management is achieved by controlling the fan, which depends on the temperature of the CPU.  The system may also be shutdown whenever the temperature can’t be kept in safe region by active cooling methods is activated.

Please refer to thermal specification and keyboard BIOS specification for the current regions of the temperature defined for thermal management.

## Power Management

### Introduction

The notebook supports ACPI power management modes. The system will dynamically switch to ACPI mode for configuration and power management when an ACPI OS is loaded.

When ACPI is not loaded and enabled, the power management function will be disabled.

### System Time-outs

The system Time-outs include the Standby time-out, the Suspend time-out, and the Hibernate time-out.

System Time-outs are handled by the operating system in ACPI mode.  BIOS time-outs must be disabled.  System time-outs are set using the control panel power applet.

### System Power Management

The overall system can be in one of five system power states as described below:

| ACPI mode | Power Management |
| --- | --- |
| Mech. Off (G3) | All devices in the system are turned off completely. |
| Soft Off (G2/S5) | OS initiated shutdown. All devices in the system are turned off completely. |
| Working (G0/S0) | Individual devices such as the CPU and hard disk may be power managed in this state. |
| - S3 Sleeping State | - CPU set power down - VGA Suspend - PCMCIA Suspend - Audio Power Down - Hard Disk Power Down - CD-ROM Power Down |
| - S4 Sleeping State | Also called Hibernate state. System Saves all system states and data onto disk prior to power off the whole system. |

- Note:  The S1, S2 will not be support on this product.

### Device Power Management

The device specific power management supported by this notebook includes the CPU and the hard disk.

### CPU Power Management

- ACPI mode

The operating system detects when the system is idle and places the CPU in one of the  CPU low power states (C1, C2,C3 or C4)depending on how much latency it believes the system can afford.

The C1 state is simply the CPU halt instruction. The C2 state is the CPU stop grant state.  The C3 state is the CPU stop clock state. The CPU stays in this state until an interrupt occurs.

### System Wake Up Sources

The table below lists the wake up events for all low power states:

#### With AC Adapter

| Events | S3 | S4 | S5 |
| --- | --- | --- | --- |
| Any Key (Internal Keyboard) | x | x | x |
| Power button | ○ | ○ | ○ |
| Lid Open | ○ | ○ | x |
| Build- in Touch Pad | x | x | x |
| Modem Ring (Internal Modem Only) | ○ | x | x |
| LAN (Onboard) | ○ | ○ | ○ |
| RTC | ○ | ○ | x |
| USB device (device plug in or plug out device) (note #1) | ○ | x | x |

#### With Battery Only

| Events | S3 | S4 | S5 |
| --- | --- | --- | --- |
| Any Key (Internal Keyboard) | x | x | x |
| Power button | ○ | ○ | ○ |
| Lid open | ○ | x | x |
| Build-in Touch Pad | x | x | x |
| Modem Ring (Internal Modem Only) | ○ | x | x |
| LAN (Onboard) | ○ | x | x |
| RTC | ○ | x | x |
| Critical low battery (Note #2 ) | ○ | x | x |
| USB device (device plug in or plug out device) (note #1) | ○ | x | x |

Mobile: Wake-On-LAN support is NOT required.

Note #1: Driver Dependent

Note #2: This event should wake the system to full ON mode then notify the OS of the Battery Critical Low event.

#### Power Button

The power button will wake the system from any low power state as described in the Power Button section.

#### Modem Ring

PCMCIA/USB Modem

- The function of waking up the system from standby is not supported.

Internal Modem

- Activity on the modem ring line will wake the system from S1/S3 (standby) if OS enable modem ring wake up.

#### LAN

PCMCIA/USB LAN

- The function of waking up the system from S1~S5 is not supported.

Onboard LAN

Onboard LAN can wake the system from S3 (standby) if OS enable wake up LAN.

Wake-On-LAN by Magic packet needs to be supported when the system is in S4/S5

and AC adapter is plugged.

#### Real Time Clock Alarm

The Real Time Clock alarm interrupt will wake the system from standby, suspend.

#### PCI PME# Signal

The PCI PME# signal can wake the system up in ACPI mode but cannot wake up the system in APM modes.

#### USB Device

If USB driver is loaded, the driver will wake the system from S1 (standby).  Since current USB drivers still don’t support the capability to wake up from S3 (standby), The actual implementation of this feature will be dependent with the driver status.

#### Critical Low Battery

Critical low battery event can’t wake the system from S3 (standby) in ACPI mode.

## ACPI (Adanced Configuration and Power Interface)

### Introductions

The Advanced Configuration and Power Interface (ACPI) is a well-specified power management and configuration mechanism.  It evolves the existing collection of power management codes, APM, PnP BIOS, ..etc.

### ACPI Sleep Status

BIOS must support the following sleep states – S3, S4 and S5.

### Fast Boot and Fast Resume

BIOS must hands off the control to the operating system within the following time limits:

|  | Required | Preferred will be required on 06/01/2006) |
| --- | --- | --- |
| S5->S0 | 10sec | 7sec |
| S4->S0 | 10sec | 7sec |
| S3->S0 | 500msec | 250msec |

*Measured using the Microsoft BootVis tool.

In addition, total resume time from S3 must be completed within 5 seconds.

### Power State Transition Diagram

- The state transition diagram in ACPI mode is as follows:

| From (State) | Leave By Condition | Enter (State) |
| --- | --- | --- |
| S3 | Power Button | S0 |
|  | Modem ring |  |
|  | On-Board LAN |  |
|  | RTC Alarm |  |
|  | USB Mouse/Keyboard devices |  |
| S4 | Resume button request |  |
| S0 | Press power button (depends on ACPI OS setting) | S3 |
|  | Press lid switch (depends on ACPI OS setting) |  |
|  | Standby icon in shutdown menu in Windows. |  |
|  | ACPI OS timer expired |  |
|  | Critical low battery (depends on ACPI OS setting) |  |
| S0 | Critical low battery (depends on ACPI OS setting) | S4 |
|  | Press lid switch (depends on ACPI OS setting) |  |
|  | Press power button (depends on ACPI OS setting) |  |

### Time-outs while On AC Power

OS determines the behavior of the feature.

### Embedded controller

The keyboard controller will act as the ACPI embedded controller and support the ACPI EC protocol and interface.

### SCI/SMI events

All ACPI OS controllable events will be triggered as SCI.  Otherwise SMI will be triggered.

## PC2001

The notebook must meet Microsoft Logo requirements in accordance with the PC2001 Design Guide and the Microsoft Logo test programs.

## Miscellaneous Features

### Single BIOS ROM

The system BIOS and Keyboard BIOS share one single flash ROM.  The size of the flash ROM is 1024KByte.

### USB Support

This feature allows the use of a USB keyboard to access BIOS Setup and to be used in DOS without additional drivers. USB floppy boot and Crisis Recovery from USB floppy is also supported.

Boot from USB Mass Storage is supported if this feature is enabled.

### Flash Utility – One BIOS ROM only

BIOS flash update programs must be provided to end-user.

Two types of flash programs are required as follows.

| OS | Media | Note |
| --- | --- | --- |
| Windows XP | Windows Application |  |
| DOS | - Bootable Diskette Image (self-extractive) - ISO CDROM image (Desktop systems only) |  |

It is preferred that the while the BIOS/EC is being flashed, power button and sleep buttons (for both S3 and S4) are disabled.

For Mobile, it is preferred that Flash Update program checks if AC adapter is plugged and remaining battery capacity is enough to avoid unexpected shutdown during flashing BIOS/EC.

Note: The flash update program must check the BIOS identifier (in the BIOS version information) that is present on the system and compare it to the identifier in the update .bin file. If they do not match, the program should terminate with and error message and not attempt to update the system flash.

All flash utilities should meet the following requirements:

- Identify the system and does not update the BIOS if the system is not supported by the BIOS.

- Need to check AC power source before starting flashing BIOS.

- If AC power source is absent, ask user to apply AC power source.

- No need to check password.

- The flash utility can be used to program both system and keyboard BIOS at the same time.

- WinFlash utility is provided to flash BIOS under Windows environment.

- WinFlash utility can self-execute and contains BIOS data for flashing.

- One single DOS mode flash utility to flash all SKUs.

- One single WinFlash utility to flash all SKUs.

### Crisis Recovery

This feature provides an opportunity for system that cannot boot up. With a crisis floppy disk, the system can be performing crisis recovery by using internal PS2 keyboard.

To perform crisis recovery using keyboard, do the following:

- Power off the system.

- Plug-in the USB floppy drive with crisis floppy inserted.

- Plug-in AC adapter and make sure it is powered.

- Power on the system from off state (i.e. cold boot) while holding down <Fn+B> key.

- After POST, release <Fn+B> key. The system should boot from floppy and perform crisis recovery action.

### EEPROM

There is one EEPROM which is used to store many important system and user data in the notebook.  The size of the EEPROM is 2K bytes. Refer to Appendix A for requirements.

### VGA Support

This section describes the expected behavior when a video monitor is connected to the VGA port on the notebook or port replicator.

The BIOS will use both of the RGB and pin 11 methods to determine the presence of an external VGA monitor.  Either case meet will indicate an external VGA monitor is present.

### TVT (ThinkVantage Technology) Boot Support

BIOS shall support for booting from TVT partition (Rescue and Recovery (R&R)) application.

Refer to Appendix B for requirements.

### Wireless LAN Device Lock and Mutual Authentication

To prevent unsupported Wireless LAN installed and run on the system, BIOS must have a mechanism to prevent it from working by checking the PCI IDs of the devices (BIOS Lock).

In addition, the mutual authentication with wireless device driver is required (Mutual Authentication).

Refer to Appendix C for the detailed requirements.

### Device Presence Detection

BIOS needs implement a SMBIOS structure (Type 135) to inform other software if certain devices are installed (present) in the system. This is, for example, to be used by preload process to decide if the software for each device needs to be installed or not.

Currently, presence detection of the following devices needs to be implemented.

- Fingerprint Reader

Refer to Appendix D for the detailed requirements.

## BIOS Version and Resource Allocated

### BIOS version number

Below lists the BIOS version numbers that will be assigned to this platform.

| Platform | QA Releases | Production Release |
| --- | --- | --- |
| HEL81 | EL80003A (0.03A) | EL80100A (1.00A) |
| HEL81 | EL80003A (0.03A) | EL80100A (1.00A) |

NOTE: xx starts at 0 and is incremented with each release of the specific type.

### Configuration Requirements

The table below lists the possible usage of the system resources:

| IRQ | Hardware |
| --- | --- |
| 00 | System Timer (High Precision Event Timer) |
| 01 | Standard PS/2 Keyboard |
| 02 | Programmable Interrupt Controller (Free if used the APIC mode) |
| 03 | Free |
| 04 | Free |
| 05 | Free |
| 06 | Free |
| 07 | Free |
| 08 | Real Time Clock (High Precision Event Timer) |
| 09 | System Control Interrupter (For SCI under ACPI mode) |
| 10 | SM Bus Controller |
| 11 | Free |
| 12 | PS/2 Compatible Mouse |
| 13 | Numeric data processor |
| 14 | Primary IDE controller (Hard Disk) |
| 15 | Secondary IDE controller (CD ROM) |
| 16 | PC Card Controller / PEG Device / USB Host Controller / PCI-Express Root Port |
| 17 | PCI-Express Root Port / PCIe Wireless LAN |
| 18 | Intel USB Universal Host Controller |
| 19 | Intel USB Universal Host Controller |
| 20 | Free |
| 21 | Realtek LAN Controller |
| 22 | Intel High definition Audio / IEEE-1394 Controller |
| 23 | Intel USB 2.0 EHCI / USB2.0 controller / 4 in 1 Card Reader |

| DMA | Hardware |
| --- | --- |
| 00 | Free |
| 01 | Free |
| 02 | Free |
| 03 | Free |
| 04 | Direct memory access controller |

### System Management BIOS(SM BIOS) version 2.4 or greater

This product require that SMB 2.4 BIOS sub-structures be supported as follows:

| TYPE | Structure Type | Required? | Content |
| --- | --- | --- | --- |
| 0 | BIOS Information | YES |  |
| 1 | System Information (Component ID) | YES |  |
| 2 | Motherboard Information | YES |  |
| 3 | System Enclosure | YES |  |
| 4 | Processor Information | YES |  |
| 5 | Memory Controller | No |  |
| 6 | Memory Information | YES |  |
| 7 | Cache Information | YES |  |
| 8 | Port Connector Information | YES |  |
| 9 | System Slots | YES |  |
| 10 | On Board Devices | YES |  |
| 11 | OEM Strings | YES |  |
| 12 | System Configuration | YES |  |
| 13 | BIOS Language Information | NO |  |
| 14 | Group Associations | NO |  |
| 15 | System Event Log | NO |  |
| 16 | Physical Memory Array | YES |  |
| 17 | Memory Devices | YES |  |
| 18 | Memory Error Information | NO |  |
| 19 | Memory Array Mapped Address | No |  |
| 20 | Memory Device Mapped Address | No |  |
| 21 | Built-in Pointing Device | YES |  |
| 22 | Portable Battery | YES |  |
| 23 | System Reset | NO |  |
| 24 | Hardware Security | NO |  |
| 25 | System Power Control | NO |  |
| 26 | Voltage Probe | NO |  |
| 27 | Cooling Device | NO |  |
| 28 | Temperature Probe | NO |  |
| 29 | Electrical Current Probe | NO |  |
| 30 | Out-of-Band Remote Access | NO |  |
| 31 | BIS, Boot Integrity Service | NO |  |
| 32 | BIOS Integrity Service | NO |  |
| 126 | Inactive | NO |  |
| 127 | End-of-Table | YES |  |

| Structure | Field | Requirement | Note |
| --- | --- | --- | --- |
| Type 0 (BIOS Information) | Offset 04h Vendor | TBD |  |
| Type 0 (BIOS Information) | Offset 05h BIOS Version | Mobile: EL800xxA | “xx” BIOS ID number (Supplied by Lenovo) “nn” BIOS release number <MUST BE DISPLAYED IN BIOS SETUP> |
| Type 0 (BIOS Information) | Offset 0Ah (BIOS Characteristics, System Vendor) | Bit48:50 : 3bit value = Enclosure type 00h = "Old Lenovo PC not supporting the new SMBIOS fields) 01h = Desktop 02h = Portable Bit51:55 : 5bit value = Brand type 00h = Reserved 01h = Lenovo 02h = Think 03h - 1Fh = reserved Bit56:63 : reserved all other are reserved | The other data in this field follows the SMBIOS standard definitions. |
| Type 0 (BIOS Information) | Offset 15h (System BIOS Minor Release) | Same as “nn” (BIOS release number) in BIOS Version | . |
| Type 1 (System Information) | Offset 04h Manufacturer | TBD |  |
| Type 1 (System Information) | Offset 05h Product Name | Must be 7digits MTM (Machine Type and Model) TBD Default = “INVALID” | See “SMBIOS Utility” below. <MUST BE DISPLAYED IN BIOS SETUP> |
| Type 1 (System Information) | Offset 06h Version | TBD | See “SMBIOS Utility” below. |
| Type 1 (System Information) | Offset 07h Serial Number | 7 digits. Default = “INVALID” | <MUST BE DISPLAYED IN BIOS SETUP> |
| Type 2 (Base Board Information) | Manufacturer | TBD |  |
| Type 3 (System Enclosure Information) | Offset 05h Type | 03h – Desktop 0Ah - Notebook |  |

In addition to the standard SMBIOS structures required by SMBIOS specification, following SMBIOS structure must be implemented for supporting TVT features.

SMBIOS Type 131: TVT Enablement

| Offset | Name | Content | Example |
| --- | --- | --- | --- |
| 00h (Byte) | Type | 131 (83h) |  |
| 01h (Byte) | Length | 16h |  |
| 02h (Word) | Handle | Varies |  |
| 04h (Byte) | Version | 01h |  |
| 05h (16 Bytes) | TVT Structure | 00h | A value of 0 implies feature available. A value of 1 implies disabled feature. By default all feature bits will be 0. Bit 0-127: TVT Features Bit 127: Diagnostics (PC Doctor) Bit 126-0: reserved |
| 15h (Byte) | Description String | 01h | “TVT-Enablement” |

This product require that the Serial Number field, which is a string at offset 7 of the Type 1 (System Information) sub-structure, is to be filled in with the units Electronic Serial Number.  This would be filled in at boot time.

### POST summary screen

The Post summary screen is a screen that appears at the end of the POST processing if quiet boot is disabled or the user presses the Esc key during POST.  The screen must contain the information listing below:

- A copyright message

- The BIOS revision number and model name in customer format

At the bottom of the screen, the screen should have the following messages:

Press <F1> to enter setup, Press <F12> to change boot device

### CMOS RAM management

The BIOS will automatically update certain information in CMOS on each boot.  This information includes:

- DRAM size and configuration

- Hard disk configuration

If the CMOS RAM fails checksum or a power lost on CMOS battery is detected during boot, an appropriate error message will be displayed:

System CMOS checksum bad – Default configuration used

The system BIOS must automatically load default values defined in the setup menu during POST when encounter these problems.  The user must not be required to take any action to continue the rest of POST(or entering SETUP).

### Network Boot

This feature allows the system to boot off of a LAN when the hard disk is absent or has not been loaded with the operating system.  It is utilized by the  software download process in manufacturing.  This product will use PXE since this is a PC2001 requirement.

PXE ROM should not be loaded if Network boot is not required. The display of the logo screen must not be interrupted even if PXE ROM displays the message.

BIOS ROM including option ROM for LAN controller on motherboard is required.

### SLP(System Locked Pre-installation) for Windows XP support

In the address range E000:0000 to F000:FFF0, the following string must be imbedded.

“LENOVO” the hex equivalent is:  4C 45 4E 4F 56 4F

To support legacy manufacturing tools, the following string must also be present in the E000:0000 to F000:FFF0 address range:

‘IBM CORPORATION’ the hex equivalent is: 49 42 4D 20 43 4F 52 50 4F 52 41 54 49 4F 4E.

The requirement for the IBM string will be removed in the future. Also, the IBM string must <not> appear in the SMBIOS structures.

### Boot Device Support and Boot Device Sequence

Following boot devices need to be supported.

- ATA Hard Disk Drive(s)

- ATAPI Optical Drive

- External USB Floppy Disk Drive

- PXE Network Boot

- USB Memory Key

- (External USB CD-ROM is preferred)

BIOS will provide the user with the ability to select the devices for attempting to boot an OS. The user will be able to specify which system device will be attempted 1st, 2nd, and so on. The list of possible boot devices will include:

- Hard Drive

- CD-ROM/DVD-ROM

- Removable Drive

- Network Boot

If a specified device (drive or media) is not currently available in the system or is not in a bootable state, the system will proceed to the next user selection device. If none of the selected devices are present or in a bootable condition then the system will halt and display an error message.

Support USB FDD bootable function and support Boot Block from USB FDD.

Pressing the F12 key during POST will allow the user to boot an alternative boot device.

As defined in Appendix C in the BIOS Boot Specification, Version 1.01, January 11, 1996, boot menu will appear when <Hot Key> is pressed. F12 is assigned for the <Hot Key>

2.10 System Setup

### Invoking setup

The setup function can only be invoked by pressing F1 when  Press <F1> to enter Setup message is prompted on the bottom of screen during POST.

The setup uses a menu driven interface to allow the user to configure their system. The features are divided into 6 parts as follows:

Main		Allows the user to specify standard system parameters.

Advanced 	Provides advanced settings of the system.

Security		Provides security settings of the system.

Boot		Allows the user to specify the boot options.

Exit			Allows the user to save CMOS setting and exit Setup.

During setup, all Fn function keys and power saving functions are disabled.

### Setup screens

#### Main Menu

| Phoenix BIOS Setup Utility |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Main |  | Advanced |  | Security | Boot |  |  | Exit |
|  |  |  |  |  |  | Item Specific Help |  |  |
| System Time: |  |  | [09:00:00] |  |  |  |  |  |
| System Date: |  |  | [03/15/2006] |  |  | <Tab>, <Shift-Tab>, or |  |  |
|  |  |  |  |  |  | <Enter> Selects field. |  |  |
| BIOS Version: |  |  | EL80003A(0.03A) |  |  |  |  |  |
| CPU Type: |  |  | Intel® Pentium® M Processor |  |  |  |  |  |
| CPU Speed: |  |  | 2.00GHz |  |  |  |  |  |
| Internal Hard Disk: |  |  | ST98823AS – (S1) |  |  |  |  |  |
| DVD/CD-ROM Drive: |  |  | TSSTcorp CDW/DVD TS-L462C |  |  |  |  |  |
| Product Name: |  |  |  |  |  |  |  |  |
| Serial Number: |  |  | xxxxxxxxxxxxxxxxxxxxxxxxxxx |  |  |  |  |  |
|  |  |  | xxxxxxxxxxxxxxxxxxxxxxxxxxx |  |  |  |  |  |
| UUID Number: |  |  | xxxxxxxxxxxxxxxxxxxxxxxxxxx |  |  |  |  |  |
|  |  |  | xxxxxxxxxxxxxxxxxxxxxxxxxxx |  |  |  |  |  |
| System Memory |  |  | 640 KB |  |  |  |  |  |
| Extended Memory |  |  | 511 MB |  |  |  |  |  |
| F1 Help | ¯ Select Item |  |  | F5/F6 Change Values |  |  | F9 Setup Defaults |  |
| Esc Exit | ¬® Select Menu |  |  | Enter Select4Sub-Menu |  |  | F10 Save and Exit |  |

System Time and System Date:

The hour is displayed with 24 hour format. The values set in these two fields take effect immediately. When CMOS is broken, default value is 01/01/2005.

Internal Hard Disk:

The hard disk types is auto detected and set by the system. If there is no hard disk present or unknown type, None should be shown on this field, otherwise the model name must be shown.

DVD/CD-ROM Drive:

The Optical Disc Drive type is auto detected and set by the system. If there is no ODD module present or unknown type, None should be shown on this field, otherwise the model name must be shown.

UUID: UUID stards for Universal Uniqe Identification.

NOTE: The sub-items under each device will not be shown if the device control is set to disable or auto.  This is because the user is not allowed to control the settings in these cases.

      - Advanced

| Phoenix BIOS Setup Utility |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Main |  | Advanced |  | Security | Boot |  |  | Exit |
|  |  |  |  |  |  | Item Specific Help |  |  |
| Quiet Boot: |  |  | [Enabled] |  |  |  |  |  |
|  |  |  |  |  |  | <Tab>, <Shift-Tab>, or |  |  |
| Boot Display Device: |  |  | [LCD mode] |  |  | <Enter> Selects field. |  |  |
| Internal Touchpad: |  |  | [Enabled] |  |  |  |  |  |
| Internal Modem |  |  | [Enabled] |  |  |  |  |  |
| F1 Help | ¯ Select Item |  |  | F5/F6 Change Values |  |  | F9 Setup Defaults |  |
| Esc Exit | ¬® Select Menu |  |  | Enter Select4Sub-Menu |  |  | F10 Save and Exit |  |

Quiet Boot:

Enabled:  Customer Logo is displayed, and Summary Screen is disabledles

Disabled:  Customer Logo is not displayed, and Summary Screen is enabled.

Boot Display Device:

- Auto:  During power on process, the system will detect if any display device is connected on external video port.  If any external display device is connected, the power on display will be in CRT (or projector) only mode. Otherwise it will be in LCD only mode.

LCD Mode:  Output images on LCD screen only

CRT Mode:  Output images on CRT screen only.

- Internal Touchpad :

Enables or Disables the build-in Touchpad device.

Enabled:  The build-in pointing device is enabled.

Disabled:  The build-in pointing device is disabled.

- Internal Modem :

Enabor Disables the build-in modem device.

Enabled:  The build-in modem device is enabled.

Disabled:  The build-in modem device is disabled.

      - Security Menu

The following is Security menu if both of password is disabled, or enter Supervisor password when password is enabled:

| Phoenix BIOS Setup Utility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Main |  | Advanced |  |  | Security | Boot |  |  | Exit |
|  |  |  |  |  |  |  | Item Specific Help |  |  |
| Supervisor Password is |  |  | Clear |  |  |  | Supervisor Password controls |  |  |
| User Password is |  |  | Clear |  |  |  | access to the setup utility |  |  |
| Set Supervisor Password |  |  | [Enter] |  |  |  |  |  |  |
| Set User Password |  |  | [Enter] |  |  |  |  |  |  |
| Hard Disk Password is: |  |  | Clear |  |  |  |  |  |  |
| Built-in HDD Password Select |  |  | [User Only] |  |  |  |  |  |  |
| Set HDD Master Password |  |  | [Enter] |  |  |  |  |  |  |
| Set HDD User Password |  |  | [Enter] |  |  |  |  |  |  |
| Password on Boot |  |  | [Disabled] |  |  |  |  |  |  |
| F1 Help | ¯ Select Item |  |  | F5/F6 Change Values |  |  |  | F9 Setup Defaults |  |
| Esc Exit | ¬® Select Menu |  |  | Enter Select4Sub-Menu |  |  |  | F10 Save and Exit |  |

- Supervisor Password is / User Password is:

- These two fields shows that Supervisor/User Passwords are set or not.

Set		System Password is set.

Clear		System Password is not set.

- Set Supervisor Password / Set User Password

Enter	This field always shows the message.

While these fields are highlighted and press Enter, a window similar to the following is shown:

Set Supervisor Password

Enter New Password [ ]

Confirm New Password [ ]

If there is an old password then setup will prompt with the following window instead and a current password will be required to be entered at first:

Set Supervisor Password

Enter current password [ ]

Enter New Password [ ]

Confirm New Password [ ]

User can now type password in field Enter New Password, and re-enter password in field  Confirm New Password  for verification.

If the verification is OK:

Setup Notice

Changes have been saved.

[ continue]

- The password setting is complete after user presses enter.

If the current password entered does not match the actual current password:

Setup Warning

Invalid password

Re-enter Password

[ continue]

If the new password and confirm new password strings do not match:

Setup Warning

Password do not match

Re-enter Password

The format of the password is as follows:

Length		No more than 8 characters.

Characters	0-9, A-Z (not case sensitive)

- Hard Disk Password is:

- This field shows that Hard Disk Password is set or not.

Set		Hard Disk Password is set.

Clear		Hard Disk Password is not set.

- Password on Boot

Defines whether a password is required or not while the events defined in this group happened. The following sub-options are all requires the Supervisor password for changes and should be grayed out if the user password was used to enter setup.

Allows the user to specify whether or not a password is required to boot.

Option : Disabled/Enabled

      - Boot Menu

This menu allows the user to decide the order of boot devices to load the operating system.  Bootable devices includes the diskette drive in module bay, the onboard hard disk drive and the CD-ROM in module bay.

| Phoenix BIOS Setup Utility |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Main |  | Advanced | Security | Boot |  | Exit |
|  |  |  |  | Item Specific Help |  |  |
| Boot priority order: |  |  |  |  |  |  |
| 1: USB FDD: |  |  |  | Use these keys to set |  |  |
| 2: USB KEY: |  |  |  | the boot order that |  |  |
| 3: ATAPI CD: HL-DT-STCD-RW/DVD DRIVE |  |  |  | the BIOS will use to |  |  |
| 4: IDE HDD0: ST98823AS – (S1) |  |  |  | attempt to boot an OS: |  |  |
| 5: PCI LAN: Realtek Boot Agent |  |  |  | <F6> and <F5> moves |  |  |
| 6: |  |  |  | the device up or down. |  |  |
| 7: |  |  |  | <x> exclude or include |  |  |
| 8: |  |  |  | the device to boot. |  |  |
|  |  |  |  | <1> Loads default |  |  |
| Excluded from boot order: |  |  |  | boot sequence. |  |  |
| : USB HDD: |  |  |  | Legacy USB support must |  |  |
| : USB CD: |  |  |  | be enabled for USB boot. |  |  |
| F1 Help | ¯ Select Item |  | F5/F6 Change Values |  | F9 Setup Defaults |  |
| Esc Exit | ¬® Select Menu |  | Enter Select4Sub-Menu |  | F10 Save and Exit |  |

      - Exit Menu

| Phoenix BIOS Setup Utility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Main |  | Advanced |  |  | Security |  | Boot |  | Exit |
|  |  |  |  |  |  | Item Specific Help |  |  |  |
| Exit Saving Changes |  |  |  |  |  |  |  |  |  |
| Exit Discarding Changes |  |  |  |  |  | Exit System Setup and save |  |  |  |
| Load Setup Defaults |  |  |  |  |  | your changes to CMOS |  |  |  |
| Discard changes |  |  |  |  |  |  |  |  |  |
| Save changes |  |  |  |  |  |  |  |  |  |
| F1 Help | ¯ Select Item |  |  | F5/F6 Change Values |  |  |  | F9 Setup Defaults |  |
| Esc Exit | ¬® Select Menu |  |  | Enter Select4Sub-Menu |  |  |  | F10 Save and Exit |  |

- Exit Saving Changes

Allows the user to save changes to CMOS and reboot the system. The following message is shown when user presses Enter on the item.

Setup Confirmation

Save configuration changes and exit now

[ Yes] [No]

- System will reboot if Yes is selected and will stay in Setup if No is selected.

- Exit Discarding Changes

Allows the user to not save changes before exiting Setup. The following message is shown when user presses Enter on this item.

Warning

Configuration has not been saved!

Save before exiting?

[Yes] [No]

- System will reboot after either selection.

- Load Setup Default

Allows the user to load default values in CMOS Setup. The following message is shown when user presses Enter on this item:

Setup Confirmation

Load default configuration now?

[ Yes] [No]

- It still stay in Setup after either selection.

- Discard Changes

Allows the user to discard previous changes in CMOS Setup. The following message is shown when user presses Enter on this item:

Setup Confirmation

Load previous configuration now?

[ Yes] [No]

- Save Changes

Allows the user to save current changes in CMOS Setup. The following message is shown when user presses Enter on this item:

Setup Confirmation

Save configuration changes now?

[ Yes] [No]

## 2.11 OS Compatibility

| OS | Retail Support |
| --- | --- |
| MS-DOS/PC-Doctor | Minimal (Diagnostics and Manufacturing utilities) |
| Microsoft Windows 2000 Professional | No Support |
| Microsoft XP Home Edition w/SP2 | Full Support |
| Microsoft XP Professional Edition w/SP2 | Full Support |
| Microsoft Windows Vista (Longhorn) | Longhorn Ready |

## APPENDIX A:   SERIAL EEPROM MAP

There is one EEPROM that is used to store many important system and user data in the notebook.  The size of the EEPROM is 2K bytes.

The total capacity of EEPROM is 2048 bytes. Following is the definition of this area

| Offset | Length | Name | Comments | Initial value |
| --- | --- | --- | --- | --- |
| 00h –1Fh | 32 Byte | System Serial Number 1 | 32 bytes of Serial number. | Control by Compal manufactory |
| 20h –3Fh | 32 Byte | System Serial Number 2 | 32 bytes of Bundle number. | May not use for this platform |
| 40h –4Fh | 32 Byte | Reserved | Unused. | Fill 0xff |
| 50h –6Fh | 32 Byte | System Version | SMBIOS Type 1 offset 06h | Provide by customer |
| 70h –7Fh | 16 Byte | UUID | 16 bytes for UUID. | Control by Compal manufactory |
| 80h –9Fh | 32 Byte | Product Name (Machine Type & Model) | SMBIOS Type 1 offset 05h | Provide by customer |
| A0h –DFh | 64 Byte | Reserved | Unused | Fill 0xff |
| E0h –E7h | 8 Byte | GUID | 8 bytes for GUID | Control by Compal manufactory |
| E8h | 1 Byte | Modem Control | 1 byte for Internal modem control | 00h: Disabled / 01h: Enabled |
| E9h –EFh | 7 Byte | Reserved | Unused | Fill 0xff |
| F0h | 1 Byte | Keyboard type | Define for US/UK/JP keyboard | US: /UK: /JP: |
| F1h - FDh | 13 Byte | Keyboard BIOS Used | Reserved for EC BIOS used |  |
| FEh –FFh | 2 Byte | EEPROM initialized flag | Set to 55AAh when the EEPROM get initialized. | Set by EC BIOS |
| 200h –23Fh |  | Asset tag number | 64 bytes for DMI Type 3 | Fill 0xff |
| 280h - 3FFh |  | Reserved | Unused | Fill 0xff |
| 400h - 4FFh |  | Reserved | Unused | Fill 0xff |

* All of reserved and unuse space must be fill to FFh

## APPENDIX B:  Rescue and Recovery BIOS

### B.1  Preboot Key Control

Because of the desire to decrease boot times, the processing of keystrokes that apply to the TVT Boot Manager has been moved into the BIOS. Please note that from the BIOS point of view, a boot to the TVT Boot Manager is the same as booting to the normal boot path.  An Int 15h messages user input to the TVT Boot Manager and this input can be used to perform various TVT tasks.  If the TVT Boot Manager is not installed, the BIOS will change it’s preboot behavior and will not message keystroke behavior as it applies to TVT’s. .  If a TVT Boot Manager is installed, the BIOS will message the appropriate keystrokes and communicate this data to the TVT Boot manager.  By implementing this infrastructure, the TVT Boot Manager does not have to add delays in the boot cycle while waiting for keystrokes.

When the TVT Boot Manager calls the BIOS INT 15h interrupt, the BIOS assumes that the TVT Boot manager has been installed on the system. This signals the BIOS not to display the Consolidated Boot Menu (if the Enter key, F11 or ThinkVantage key is pressed) on the next boot.   It is assumed that the TVT Boot Manager will process these keystrokes and perform the requested function.  An internal flag (usually a non checksummed CMOS value) will be set.  This flag indicates that the TVT Boot Manager is installed on this system.  This flag is not cleared until the next time the system boots to the primary hard file.  If the TVT Boot Manager is installed, it will again issue the Preboot Int 15h that will set the flag for the next boot.

The return parameters from the Int 15h call informs the TVT Boot Manager if the F11, ThinkVantage or enter key was pressed on the current boot. If any of the keys were pressed, the TVT Boot Manager then performs the appropriate function.  The status indicator that a key was pressed will be cleared each boot cycle so no internal storage is required.

Entry:        ax - 4200h

Exit:         Carry Flag    - function not supported

No Carry Flag - function supported

- AX = 0001 - F11/Enter/ThinkVantage key pressed

- AX = 0000 - F11/Enter/ThinkVantage key not pressed

### B.2  Preboot Key Processing

| Key Stroke | Preboot Int 15h not issued | Preboot Int 15h issued |
| --- | --- | --- |
| Enter or ThinkVantage Button | B.3 Consolidated Boot Menu | Normal Boot Sequence. Boot manager will process key strokes |
| F1 | Enter ROM Based Setup | Enter ROM Based Setup |
| F11 | Normal Boot Sequence | Normal Boot Sequence. Boot manager will process key strokes |
| F12 | B.4 Boot Alternate Device Menu | B.4 Boot Alternate Device Menu |

### B.3  Consolidated Boot Menu

This menu will appear only if int 15h (ax = 4200) is not called and the user presses the ThinkVantage key or the Enter key. A 10 second countdown counter appears at the bottom of the menu and is stopped if the user presses the Enter key.

| Startup Interrupt Menu |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Press one of the following keys to continue: |  |  |  |  |
|  |  | ESC to resume normal startup |  |  |  |
|  |  | F1 to enter the BIOS Setup Utility F11 to diagnose hardware or recover your system |  |  |  |
|  |  | F12 to choose a temporary startup device |  |  |  |
|  | Press ENTER to pause ... |  |  |  |  |

### B.4  Boot Alternate Device Menu

This menu appears when the user presses the F12 key during POST. The user can then choose the boot device.

| Startup Device Menu |  |  |  |
| --- | --- | --- | --- |
| 1. |  | Diskette Drive A: |  |
| 4. |  | IDE HDD: | Vendor Name |
| 5. |  | IDE CD: | Vendor Name |
| 6. |  | PCI BEV | Vendor Name |
|  | <Enter Setup> |  |  |

## APPENDIX C: Wireless LAN Support

### BIOS Lock

POST must have a table of supported wireless LAN cards by their IDs and if unsupported devices are installed, POST must show an error message and halt.

### Mutual Authentication

In addition to the BIOS Lock by device IDs, BIOS also needs to support the mutual authentication with wireless device driver.

What BIOS needs to do is to have the following SMBIOS table.

The table should appear (become active) only when the supported Wireless LAN cards are installed.

SMBIOS Type 133: Wireless Device Authentication

| Offset | Name | Content | Description |
| --- | --- | --- | --- |
| 00h | Type | 133 (85h) |  |
| 01h | Length | 05h |  |
| 02h | Handle | Varies |  |
| 04h | OEM Data String | 01h | “KHOIHGIUCCHHII” If this string is found, wireless device driver is allowed to run on the system. |

## APPENDIX D: Device Presence Detection

### Device Presence Detection and Notification

BIOS needs implement a SMBIOS structure (Type 135) to inform other software if certain devices are installed (present) in the system. This is, for example, to be used by preload process to decide if the software for each device needs to be installed or not.

Currently, presence detection of the following devices needs to be implemented.

- Fingerprint Device (Fingerprint Reader)

SMBIOS Type 135: Bulk for Lenovo Mobile PC Unique OEM Data

| Offset | Name | Content | Description |
| --- | --- | --- | --- |
| 00h (Byte) | Type | 135 (87h) |  |
| 01h (Byte) | Length | Varies (>= 08h on current format) | Total length of the formatted area of this structure, including a child (TP OEM) structure |
| 02h (Word) | Handle | Varies |  |
| 04h (Word) | Signature | 5054h | (=’TP’) |
| 06h (Byte) | Offset to TP OEM Structure | N (=07h on current format) |  |
| 07h (Byte) | OEM Structure Number | 03h |  |
| 08h (Byte) | OEM Structure Revision | 01h for the following format |  |
| 09h (Byte) | Device Presence Bits | Bit 0: 0=No fingerprint device 1=Fingerprint device is present Bit 1-7: Reserved. Set to 0. |  |